# Template File Introduction

Even though main DFT architectures, design methodology and test access mechanisms are converged across all the projects, specific implementation of DFTs may differ from project to project. To address such deviation in the implementation of DFTs, SPF allows user to define or redefine specific tool behavior for handling certain test access mechanisms using template file while avoiding hardcoding the project specific needs. With template definition, the design specific tool customization for each project can be achieved while the standard DFT architecture being handled by default.

In addition to template information, the tool options to set different tool modes can be given in template file. Template file is given to SPF as an input commandline option with --templateFile <file name>.

## Case sensitivity

Template file keywords are case insensitive. Note however that names of transform, packet fields, etc are strictly case sensitive.

# Template File Structure

Template file mainly consists of two parts. One is transformation definition where user can define or redefine the actual transformation steps for given test access mechanism. The other is a section for tool option setting.

## Transformation Definition

Transformation definition section is mainly comprised of three parts - TEMPLATE, TRANSFORM and TRANSFORM_SEQ. A TEMPLATE defines the basic mechanism for constructing contents (or packets) at a TAM (test access mechanism) interface level from SPF test commands. TRANSFORM and TRANSFORM_SEQ define the conversion of contents from one TAM to another TAM. TRANSFORM_SEQ takes SPF test commands at source TAM as input and converts them to another SPF sequence at destination TAM level. On the other hand, TRANSFORM takes contents (packets) defined at source TAM interface level as input and converts them to contents at destination TAM which may be an SPF sequence at destination TAM or packets at destination TAM interface.

If the full chip content generation goes through multiple TAMs starting from the intent level transaction, a TEMPLATE or TRANSFORM_SEQ followed by multiple TRANSFORMs may be used for the content generation.

Different TEMPLATEs can be defined for the same TAM and user has an option to choose a TEMPLATE for a given TAM at any given point of the input test sequence. A default TEMPLATE can be defined by using DEFAULT_TEMPLATE keyword. DEFAULT_TEMPLATE will be used for a given TAM if it is defined and a specific TEMPLATE is not explicitly set by user. The same usage applies to TRANSFORM and TRANSFORM_SEQ. The keywords for default TRANSFORM and TRANSFORM_SEQ are DEFAULT_TRANSFORM and DEFAULT_TRANSFORM_SEQ, respectively.

### TEMPLATE

TEMPLATEs are defined per each test access type - TAP or test port (TEST_PORT). Each TEMPLATE is assigned with a uniquely identifiable name. The body of TEMPLATE consists of blocks of instructions used for producing fullchip test. Each instruction block is enclosed by SCOPE construct and defined per each input test command. A list of input test commands can be given for a SCOPE if the commands share the same instructions for processing. It is not allowed to list the same command under the multiple SCOPEs.

A list of parameters may be declared in the PARAMETERS section in order to make a TEMPLATE parameterized. Please note that PARAMETERS section is not allowed in DEFAULT_TEMPLATE. A parameterized TEMPLATE is explained in more details at “Parameterized TEMPLATE (TRANSFORM/TRANSFORM_SEQ)” section.

The basic structure of TEMPLATE definition is given below.

TEMPLATE|DEFAULT_TEMPLATE <test access type> <template name> {

PARAMETERS = {

comma separated list of <var declaration>;

}

SCOPE: <list of commands> = {

...

}

SCOPE: <list of commands> = {

...

}

...

}

<test access type> ::= TAP|TEST_PORT

<var declaration> ::= $<var name>[{type=string|integer|verilog_value}]

Each SCOPE is composed of input/output packet definition sub-blocks or a procedure sub-block. A procedure sub-block may be accompanied with an optional base operation sub-block. In a packet definition sub-block, the instruction to directly construct the target packet is described. Each field of the target packet is supposed to be defined individually with data from underlying resource to access, a contant value or a parameter variable (please refer to “Parameterized TEMPLATE (TRANSFORM|TRANSFORM_SEQ)” section). Any packet field undefined in the TEMPLATE definition will be filled with the default value set in the actual packet definition.

A procedure sub-block is defined with STEPS keyword. This sub-block is used in describing sequential steps required to process the given input command. Specific set of statements are defined to be used in a STEPS block per each test access type. BASIC_OPERATIONS section may be optionally defined in SCOPE as a companion block of STEPS if STEPS is defined in the SCOPE. Basic operations used in STEPS may be redefined in BASIC_OPERATIONS section. Currently, shift_dr and shift_ir statements may be redefined in BASIC_OPERATION section. The SCOPE section definition is given below.

SCOPE: <list of commands> = {

<input/output packet> | <procedure> [<base_operation>]

...

}

<input/output packet> ::=

<dest input/output packet> = {

<dest input/output packet field> = <underlying resource data> | <value> | <parameter variable>;

...

}

<procedure> ::=

STEPS = {

<stmt>|<for_loop>;

...

}

<base_operation> ::=

BASIC_OPERATION = {

<baseOpStmt> = {

<base_stmt>|<for_loop>

...

}

}

If the default behavior for constucting contents at the given TAM interface is not implemented in the tool, defining DEFAULT_TEMPLATE is mandatory and the complete set of commands used in the content generation at the given TAM should be defined under the SCOPEs of the default template. If the default behavior is already implemented in the tool, DEFAULT_TEMPLATE may define only the subset of commnads used in the content generation at the given TAM. In this case the default tool behavior will be used for the commands which are not defined in DEFAULT_TEMPLATE. If user changes to another template from the default one, the tool will pick up the direction from the new template for processing the commands that are defined in the new template. For the commands that are not defined in the given template, the tool will pick up the direction from the default template for processing them if they are defined in the default one. If there are commands that are not defined even in the default template, the default tool behavior implemented in the tool will be used. In a gvien TEMPLATE it is not allowed to define only partial transformation information for a given command in SCOPE.

### TRANSFORM

TRANSFORM definition:

TRANSFORM|DEFAULT_TRANSFORM <dest test access type> <src test access type> <func name> {

INTERFACE_REDEFINITION {

<Interface name>:

USER_DEFINED = {

INPUT = {

<Packet name>[L-1:0] = {PinNameL-1, ..., PinName0} | <bus name>[<index range>], ... | REUSE_PINGROUP;

}

OUTPUT = {

<Packet name>[L-1:0] = {PinNameL-1, ..., PinName0} | <bus name>[<index range>], ...| REUSE_PINGROUP;

}

}

}

CLOCK = {

<packet name> : <Period sequence specified in binary>;

}

DELAY = {

<src test access type>|<dest test access type> : <unsigned int>|$<var>;

}

PARAMETERS = {

comma separated list of [$<var name>[{type=[string|integer|verilog_value}]  | @<array name>];

}

ATTRIBUTES = {

@set_attribute <attribute> <options>;

}

CONST_VAR = {

$<var name> = <value>;

}

<dest packet>:<src packet> [<optional mapping name>] = {

<destination field> = <source field> | <value> | <variable>;

…

}

PRE_AMBLE = {

[<local var definition>]

<Sequential steps>;

}

STEPS = {

[<local var definition>]

<Sequential steps>;

}

POST_AMBLE  = {

[<local var definition>]

<Sequential steps>;

}

}

The format for a TRANSFORM is given in the side bar. TRANSFORMs are defined for each pair of source and destination test access types. Each TRANSFORM is assigned with a uniquely identifiable name. The TRANSFORM body consists of blocks of instructions how to construct destination packets from source packets.

TRANSFORM definition contd:

<dest test access type>|<src test access type>::= TAP|STF|TEST_PORT

<local var definition>::=

LOCAL_VAR = {

<var> = <initial value>;

}

<Sequential steps>::=<Stmt> | <for loop>

<for loop>::=

for ($<loop variable>=<initial value>; <final value>; <increment or decrement>) {

<Stmt>;

...

}

<Stmt>::= [<assignment> |<commands>| <packetize>|<directive>]

<assignment>::= [<simple assignment> | <arithmetic assignment> | <conditional assignment>]

<packetize>::=

PACKETIZE = {

<dest packet name>:<Src packet name>[::<optional name>](parameters);

...

}

<commands>::= comment, pass itpp, focus_tap, set, compare, execute, tap_raw_shift, flush

<src spec> ::= <src field> | <src packet>

<directive> ::=@set clock_toggle <packet name> [on|off];

The various blocks in the TRANSFORM are described below and can be roughly classified as transform header and body –

- The transform header contains the following:

- INTERFACE_REDEFINITION: The packet to pin mapping in the INTERFACE section defined in the collateral can be redefined in the TRANSFORM. This is currently supported in MCI to DDR transformation.

- CLOCK: This section defines the clock behavior to be applied to every vector in the itpp.

- DELAY: The delay incurred from the transaction between source access mechanism and destination access mechanism is typically given in design spec file. However, if actual delay value is different from the one given in the spec file, it can be specified in this TRANSFORM construct and this value will override the one in the spec file. DELAY may be given in terms of source access type or destination access type. For example, DELAY={MCI: 3; ddr: 1;}.

- ATTRIBUTES: ATTRIBUTES define specific modes of transform process, which are predefined in the tool. The following attributes are supported as of today.

  - mci_bubble_insertion : enable periodic bubble insertion in MCI to DDR transformation (this is supported only in MCI to DDR transformation). In this attribute bubble definition and bubble insertion rate are defined. Syntax is shown below.

@set_attribute mci_bubble_insertion mci_packet_count = <number>, mci_bubble_count = <number>, mci_bubble_value = <bin|hex|dec>;

In this mode #mci_bubble_count bubbles are inserted after every #mci_packet_count MCI packets during transformation.

- Variables: TRANSFORM header may contain the following types of variables:

  - PARAMETER: list of the parameters used in TRANSFORM. The value for the variables is specified through the @set_transform directive in test sequence. Also discussed in the – Parameterized TEMPLATE(TRANSFORM) section.

  - CONST_VAR: list of constant variables are defined. The values of the constant variables cannot be changed in TRANSFORM.

  - GLOBAL_VAR: list of global variables are defined. The initial values can be set with a value or with parameter variables or constant variables. Global variables are accessible at all the sections of TRANSFORM . The values of global variables can be changed anywhere in the transform and the changed values will be kept incrementally until the context of the SPF sequence, for which given TRANSFORM is applicable, is changed or POST_AMBLE is exercised. The variables will be reset to initial values upon the said conditions. The global variables are currently supported for MCI-to-TAP transformation.

- The transform body contains the following:

- Source to destination packet mapping: Each statement in the block defines an individual field of the destination packet with a source packet field, a constant value or a variable (TRANSFORM parameter, constant variable, global variable, map function parameter). Similar to TEMPLATE, any undefined destination field will be filled with the default value set in the actual packet definition.

  - The mappings serve as the main transformation step if an explicit main transformation block, STEPS, is not defined. If STEPS is defined as main transformation block, the mappings can be called from STEPS. In this case the mappings can be considered as subroutines and be parameterized with their input parameters.

  - In case the mappings are used as subroutines, multiple mappings can be defined for the same source/destination packet pair. In this case each mapping should have a unique map function name.

  - Both DEFAULT_TRANSFORM and TRANSFORM should have complete transformation definition from source to destination. In other words, a TRANSFORM with partial transformation knowledge is not allowed.

- Sequential blocks:

  - Supported statements in the procedural blocks PRE_AMBLE, POST_AMBLE, STEPS are - assignments, PACKETIZE, SPF commands – comment,pass,cycle, focus_tap, set, compare, tap_raw_shift, execute, flush. If the procedural blocks contain any flush block commands like set, compare, tap_raw_shift, execute, they should be terminated with a flush in the procedural block. There may be an optional focus_tap prior to the beginning of a flush block in the procedural block.

  - The sequential blocks may contain local variables that are usable only within the block defined.

  - PREAMBLE: These are the steps required before beginning any packet transformation from source to destination.

  - POST_AMBLE: These are the steps required after performing transformation of packets from source to destination.

- STEPS: These are the main transformation steps that convert the source packets to destination packets. This section may contain local variables that may be usable only within the STEPS scope.

  - PACKETIZE: This construct is used to invoke appropriate source to destination packet mappings for destination packet construction among list of the defined mappings. The section must contain mappings to create all the packets defined in the INTERFACE/INTERFACE_REDEFINITION section. If a mapping to call is parameterized, input parameter values should be passed during the map function call.

  - Directive: certain TRANSFORM specifc directives are defined to control transformation process, and they are used in the sequential blocks.

    - clock_toggle: turn on or off clock toggling

@set clock_toggle <clock packet name> on|off;

  - LOCAL_VAR section may be defined in any of the procedural blocks. These variables need to be initialized when defined.

  - Currently, the sequential blocks are only supported for MCI to DDR, MCI to TAP transformation.

### TRANSFORM_SEQ

The format for a TRANSFORM_SEQ is given below. TRANSFORM_SEQs are defined for each pair of source and destination test access types. Each TRANSFORM is assigned with a uniquely identifiable name. The TRANSFORM_SEQ body consists of blocks of instructions how to convert the source command to destination level spf sequence. Each instruction block is enclosed by SCOPE construct and defined per each input test command. A list of input test commands can be given for a SCOPE if the commands share the same method for processing. It is not allowed to list the same command under the multiple SCOPEs.

TRANSFORM definition:

TRANSFORM_SEQ|DEFAULT_TRANSFORM_SEQ <dest test access type> <src test access type|CR type> <func name> {

PARAMETERS = {

comma separated list of [$<var name>[{type=[string|integer|verilog_value}] | @<array name>];

}

CONST_VAR = {

$<var name> = <value>;

}

GLOBAL_VAR = {

$<var name> = <value>;

}

SCOPE : <list of commands> = {

STEPS = {

<local variable definition>

<Sequential steps>

}

}

}

<local var definition>::=

LOCAL_VAR = {

<var> = <initial value>;

}

<Sequential steps>::=<Stmt> | <for loop> |<if else>

<for loop>::=

for ($<loop variable>=<initial value>; <final value>; <increment or decrement>) {

<Stmt>;

...

}

if (<expression>) {

<Sequential steps>

}

else if (<expression> {

<Sequential steps>

}

else {

<Sequential steps>

}

<Stmt>::= [<assignment> |<commands>|]

<assignment>::= [<simple assignment> | <expression assignment> | <conditional assignment>]

<var> = <local_var>| <global_var> | <cr_var>

<commands>::= comment, pass itpp, focus_tap, focus_tap_rf, set, compare, execute, tap_raw_shift, flush, focus_stf, cycle

<src spec> ::= <src field> | <src packet>

<simple assignment> ::= <var> = <var_or_value>

<expression assignment> ::= <var> = <expression>

<expression>::= <var_or_value> <operators> <var_or_value> | <operators> <var_or_value>
<operators> ::= <arithmetic op> | <logical_op> | <comp_op> | <get_size_op>|

<comp_op>::= ‘==” | “!=” | “>=” | “>” | “<” | “<=”|

<logical_op> ::= “&&” | “||” | “!”

<arithmetic_op>::= “+” | “-“ | “*” | “/”|

<get_size_op>::=  get_size(<cr_var_spec>)

<var_or_value>::= <var> | <value>

<cr_var_spec>::= <cr_var> | <cr_var>[<range>]

<cr_var>::= <PRE_DEFINED_VAR> | <USER_DEFINED_VAR>

<var_spec>::= <var> | <var>[<range>]

<range>::=[<int>]|[<int>:<int>]

The various blocks in the TRANSFORM_SEQ are described below and can be roughly classified as transform header and body –

- The transform header contains the following:

- Variables: TRANSFORM_SEQ header may contain the following types of variables:

  - PARAMETER: list of the parameters used in TRANSFORM_SEQ. The value for the variables is specified through the @set_transform_seq directive in test sequence. Also discussed in the – Parameterized TEMPLATE(TRANSFORM/TRANSFORM_SEQ) section.

  - CONST_VAR: list of constant variables are defined. The values of the constant variables cannot be changed in TRANSFORM_SEQ.

  - GLOBAL_VAR: list of global variables are defined. The initial values can be set with a value or with parameter variables or constant variables. Global variables are accessible at all the sections of TRANSFORM_SEQ . The values of global variables can be changed anywhere in the transform and the changed values will be kept incrementally until the context of the SPF sequence, for which given TRANSFORM is applicable, is changed or POST_AMBLE is exercised. The variables will be reset to initial values upon the said conditions.

- The transform body contains the following:

- Sequential blocks:

  - SCOPE: Each SCOPE is composed of a procedure sub-block. The procedural sub-block is enclosed by STEPS construct. This sub-block is used in describing sequential steps required to process the given input command.

  - The sub-blocks - STEPS may contain local variables that are usable only within the STEPS block defined.

  - LOCAL_VAR section may be defined in STEPS. These variables need to be initialized when defined.

  - Supported statements in STEPS are - assignments, SPF commands – comment,pass,cycle, focus_tap,focus_tap_rf, set, compare, tap_raw_shift, execute, flush, focus_stf. If the procedural blocks contain any flush block commands like set, compare, tap_raw_shift, execute, they should be terminated with a flush in the procedural block.

  - User can query the size/width of any verilog type CR predefined or user defined properties using get_size function within the STEPS in the TRANSFORM_SEQ. The syntax for get_size is get_size(<cr_var>) , where <cr_var> is predefined or user defined CR property of verilog type. The get_size function can be part of the expression in variable assignment ,or part of if else condition expression, or part of for loop.  If the CR property is not of verilog type, then the tool will report error.

get_size(<cr_var_spec>)

<cr_var_spec>::= <cr_var> | <cr_var>[<range>]

<cr_var>::= <PRE_DEFINED_VAR> | <USER_DEFINED_VAR>

<PRE_DEFINED_VAR> ::= ‘<cr property name>

<USER_DEFINED_VAR>::= REG_FILE::<property name> | REG::<cr property name> | FIELD::<cr property name>

<range>::=[<int>]|[<int>:<int>]

Example of get_size usage:

TRANSFORM_SEQ TAP CR::IOSF_SB TAP2IOSF {

PARAMETERS = {

$TAP{ type = string;}, $NP_WRITE, $NP_OVRD_FLAG;

}

SCOPE : set = {

STEPS = {

LOCAL_VAR = {$portid_width = 0;}

$portid_width = get_size(‘PORTID) - 1;

if (get_size('PORTID) > 8 ) {

comment "setting port id greater than 8 bits";

set SBHIER_HEADER->DEST16 = 'PORTID[$portid_width:8];

}

}

}

  - Currently, TRANSFORM_SEQ is only supported for CR to TAP and CR to STF transformation.

- If the default behavior for constucting contents at the given TAM interface is not hard-coded in the tool, defining DEFAULT_TRANSFORM_SEQ is mandatory and the complete set of commands used in content generation at the given TAM should be defined under the SCOPEs of the default transform sequence.

  - If user changes to another transform_seq from the default one, the tool will pick up the behavior from the new transform_seq for the commands defined in the transform_seq. Only scopes for set/compare commands are allowed to be defined in TRANSFORM_SEQ. For undefined commands, the tool will report an error.

### Syntax of commands supported in template file

Focus Tap
Accessing multiple TAPs in series: Focus_tap <tap name>,<tap name>,…;

Accessing multiple TAPs in parallel:

- TAPs are under same parent: focus_tap <normal mode tap>,=<shadow mode tap name>,…;

- TAPs are under different parents: focus_tap =(<tap name1>,<tap name2>,..);

Set
Set <tap instruction and reg info>:= <default>|<previous>|<mnemonic>|<Verilog value>|<variable>|<src packet>

Compare
Compare <tap instruction and reg info>:= <default>|<previous>|<mnemonic>|<Verilog value>|<variable>

Flush
Flush;

Tap Raw Shift
Ir_tdi value:= <opcode name> | <Verilog value> | <variable>
Ir_tdo value:= <Verilog value> | <variable>
Dr_tdi value:= <Verilog value> | <variable> | <src packet>
Dr_tdo value:= <Verilog value> | <variable>

Cycle
cycle <number>
cycle <variable>

Execute
execute <op-code name>
execute <tap name> => <op-code name>

Pass

Pass itpp “string”;

Comment

Comment “string”;

where,

<variable>:= constant/global/parameter/local variable

<src packet>:= PACKET::MCI_IN, PACKET::MCI_IN[<number>|<variable>], PACKET::MCI_IN[<number>|<variable> :  <number>|<variable>]

<verilog value>:=binary/decimal/hexadecimal/fill value.
<tap instruction and reg info>:=<tap name> |

<tap name>[<number>] |

<tap name>[<number>:<number>] |

<tap name> => <reg name> |

<tap name> => <reg name>[<number>]  |

<tap name> => <reg name>[<number>:<number>]  |

<tap name> => <reg name> -> <alias name> |

<tap name> => <reg name> -> <alias name>[<number>:<number>]   |

<tap name> => <reg name> -> <alias name>[<number>] |

<reg name> -> <alias name> |

<reg name> -> <alias name>[<number>:<number>]   |

<reg name> -> <alias name>[<number>] ;

### Switch from TEMPLATE (TRANSFORM|TRANSFORM_SEQ) to TEMPLATE (TRANSFORM|TRANSFORM_SEQ)

The switch from one TEMPLATE (TRANSFORM) to another is readily allowed in the middle of test sequence using on-the-fly directives. Please refer to “Pattern Framework Spec - Test” for details.

### Parameterized TEMPLATE (TRANSFORM|TRANSFORM_SEQ)

In a parameterized TEMPLATE (TRANSFORM|TRANSFORM_SEQ), user can define a TEMPLATE (TRANSFORM|TRANSFORM_SEQ) using user defined TEMPLATE (TRANSFORM|TRANSFORM_SEQ) variables. User defined variables should be first declared in PARAMETERS section before used in the blocks of instructions for transformation. User defined variables which are not listed in PARAMETERS cannot be used in TEMPLATE (TRANSFORM|TRANSFORM_SEQ). All the variables declared in PARAMETERS section should be initialized in “TEMPLATE_VARIABLES” field of the directives for setting TEMPLATE (TRANSFORM|TRANSFORM_SEQ)  in test sequence - @set_template or @set_transform or @set_transform_seq. Optionally, the user can specify the type for single variables, which can be one of string, integer or verilog_value. By default, the variables are treated as type verilog_value. The variables defined in “TEMPLATE_VARIABLES” should match the ones declared under PARAMETERS section one to one. Please refer to “Template and Transform Control directive” section in “Pattern Framework Spec - Test” for detailed information about TEMPLATE (TRANSFORM|TRANSFORM_SEQ) setting.

Since the user defined variables in PARAMETERS are used as parameters passed by user, their values cannot be initialized or changed in TEMPLATE (TRANSFORM|TRANSFORM_SEQ). The user defined TEMPLATE (TRANSFORM|TRANSFORM_SEQ) variables can be used wherever constant value assignment or specification is allowed as shown in previous sections.

Both scalar type variable ($<var name>) and array type variable (@<var name>) are supported. Array type variables are declared without the size information in PARAMETERS section. The size of an array variable is determined when the array is initialized in the directive for setting the TEMPLATE (TRANSFORM|TRANSFORM_SEQ) in test sequence. If the size of an array needs to be used in TEMPLATE (TRANSFORM|TRANSFORM_SEQ), $#<array name> operator can be used. The $# operator of an array may be used only in “for” statement of a for loop at the moment and while indexing an array. A specific element of an array variable may be accessed with proper index set - $<array name>[<index>]. The index of an array may be set using an arithmetic expression in terms of loop variable or array size. Please refer to “For loop support” section for details about for loop support in TEMPLATE.

## Transformation with Use of Template

### TAP

TEMPLATE(s) are used to define post shift IR/DR states, to support SWCOMP and to add additional commands - cycle and pass. Basic operations can be redefined for different commands in the same template.

#### PARAMETERS declaration

Parameters are declared in this section and are used in the SCOPE definitions. The parameters can be scalar variables or array variables.

#### SCOPE definition

SCOPEs can be defined for the following set of TAP commands.

set : input test sequence command

compare : input test sequence command

tap_raw_shift_irdr : the SCOPE for tap_raw_shift command when both ir and dr sequences are specified in the command

tap_raw_shift_ir : the SCOPE for tap_raw_shift command when only ir sequence is specified in the command

execute : input test sequence command

focus_tap : input test sequence command

unfocus_tap : input test sequence command

TEMPLATE definition:

DEFAULT_TEMPLATE|TEMPLATE <test access type> <template name>

{

PARAMETERS = {

comma separated list of [$<var name> [{type=[string|integer|verilog_value}] | @<array name>];

}

...

SCOPE : <set, compare, tap_raw_shift_irdr> = {

STEPS = {

<TAP_stmt>|<for_loop>

...

}

BASIC_OPERATION = {

shift_dr = {

<base_stmt>|<for_loop>

...

}

}

}

...

}

<TAP_stmt> ::= shift_ir | shift_dr | link_ir | link_dr | <to_state> | pass | cycle

<base_stmt> ::= <shift_stmt> | <to_state> | cycle

<shift_stmt> ::= shift_by <value> | <template variable>;

<to_state> ::= to_state = “<Tap state>” ;

STEPS is defined in a SCOPE to describe procedural steps required to process each command. Some of the basic statements used in STEPS may be redefined in BASIC_OPERATION section when needed.

##### STEPS definition

The statements and their syntax allowed in STEPS in TAP context are given below.

shift_ir  : IR sequence

shift_dr : DR sequence

link_ir  :  link IR sequence in case of tap link. Note that in case of tap link, if the transactions are in the top level tap like the cltap, the same template may be used and the link_ir defined in the STEPS of the template  will be skipped

link_dr : link DR sequence in case of tap link. Note that in case of tap link, if the transactions are in the top level tap like the cltap, the same template may be used and the link_dr defined in the STEPS of the template  will be skipped

to_state = “<TAP FSM state>” : TAP FSM state after shift IR/DR

pass <target> “<string>” : the same as input test sequence command

cycle <count | template variable> : the same as input test sequence command

When a command which belongs to a SCOPE is given in the input test sequence and is processed by the tool, the STEPS defined under the SCOPE will replace the default tool behavior.

##### BASIC_OPERATION definition

BASIC_OPERATION section is defined per each SCOPE such that basic operations can be redefined for different commands in the same template. Currently, shift_dr is the only operation supported in BASIC_OPERATION section. The statements and their syntax allowed in BASIC_OPERATION in TAP context are given below.

shift_by <dr shift length | template variable> : User can break the overall shift into multiple segments of smaller shifts by calling “shift_by”. A shift count for a smaller shifts is specified for each “shift_by” statement. A separate “scand” sequence with the length of the specified value will be produced. The total count of each shift given by “shift_by” should be the same as the overall dr shift required. A predefined variable ‘REMAINING_DR_SHIFT, which represents the number of remaining shifts after counting the total number of shifts called by previous “shift_by” statements, can be used in setting “shift_by” statement.

to_state = “<TAP FSM state>” : the state after each smaller shift can be specified using to_state.

cycle <count | template variable> : the same as input test sequence command

##### For loop support

Loop definition:

for ($<loop variable>=<initial value>; <final value>; <increment or decrement>) {

<stmt>;

…

}

<initial value>, <final value>, <increment or decrement> = arithmetic function of template variables and sizes of template array variables with operators, ‘+’, ‘-‘ and ‘*’.

If the loop is used in STEPS, <stmt> = <TAP_stmt>.

If the loop is used in shift_dr, <stmt> = <base_stmt>.

User can write a loop around multiple statements in STEPS and in base operations defined in BASIC_OPERATION. The structure of a for loop is defined in the side bar. “for” statement consists of three components. The first component is the definition of a loop variable, where the intial value of the variable is set. <initial value> can be an integer or an arithmetic function of template variables and size of an array variable (with ‘+’, ‘-‘, ‘*’). The loop variable may be accessed only within the given loop. The second component is the final value of the loop variable. <final value> can be specified in the same way as <initial value>. The third component is the increment or decrement of the loop variable. <increment or decrement> can be specified in the same way as <initial value>. The evaluated value of <increment or decrement> should not be 0.

The iteration of a for loop should be always controlled with the value of the loop variable defined in “for” statement. At the first iteration, the loop variable will be set to <initial value>. At the next iteration, the loop variable will be changed by <increment or decrement>. At each iteration, if the value of the loop variable is qualified within the range, the statements inside the loop are exercised. Otherwise, the flow will exit the loop without exercising the statements inside the loop.

The followings are qualifying conditions for exercising the statements inside a loop at each iteration.

- In case  <increment or decrement> is a positive value, the value of the loop variable is equal to or smaller than <final value>.

- In case <increment or decrement> is a negative value, the value of the loop variable is equal to or larger than <final value>.

If <increment or decrement> is a positive value and <final value> is smaller than <initial value>, no iteration of the for loop will be exercised. Similarly, if <increment or decrement> is a negative value and <final value> is larger than <initial value>, no iteration of the for loop will be exercised. An index of an array variable used in a for loop can be a function of a loop variable defined in the loop or size of an array.

All the statements used in STEPS except “shift_ir”,  “shift_dr” and “to_state” are allowed in a loop used in STEPS. “shift_ir” and “shift_dr” should be given outside of a loop to guarantee the determinism of their locations and the number of their occurrences. Since to_state should be provided after a shift_ir or shift_dr in STEPS, to_state may not be specified in the loop because based on the loop variables, the loop may or may not be exercised. All the statements used in the definition of a base operation in BASIC_OPERATION section are allowed in a loop used in the base operation definition. Nested loops (a loop inside another loop) are not supported.

An example usage of for loop is for SWCOMP operation where the full-chip sequence is broken down into smaller shifts (please refer to “Pattern Framework Spec – Test” for more information).

#### Template Definition for Post IR/DR State for Commands with both Shift IR & DR

The commands producing both shift IR & DR - set, compare, tap_raw_shift with both IR & DR specification - can be combined under the same SCOPE as below.

DEFAULT_TEMPLATE|TEMPLATE TAP <template name>

{

...

SCOPE : <set, compare, tap_raw_shift_irdr> = {

STEPS = {

shift_ir;

to_state = <state> ;

[pass|cycle] ;

shift_dr ;

to_state = <state> ;

[pass|cycle] ;

}

...

}

}

The sequence of the statements in the body of the STEPS block defines how the commands would be translated to output test sequence. Therefore, whenever any of these commands occurs in the test sequence, shift_ir tells that the shift IR sequence generated by the command is first written to output test sequence file. The next step tells which TAP FSM state will be reached after the shift IR. Optionally, pass and/or cycle command(s) may be added following the IR shift. Pass and cycle are the only commands allowed in this step. Next, shift_dr tells that the shift DR sequence generated by the command is written to output test sequence file.The next is TAP FSM state after shift DR. Again, pass and/or cycle command(s) may be added after the DR shift.

#### Template Definition for Post IR State for Commands with Shift IR Only

The commands producing only shift IR - execute, tap_raw_shift with IR specification only - require separate SCOPE definition, and they can be combined under the same SCOPE as below.

DEFAULT_TEMPLATE|TEMPLATE TAP <template name>

{

...

scope : <execute, tap_raw_shift_ir> = {

steps = {

shift_ir;

to_state = <state> ;

[pass|cycle] ;

}

}

...

}

#### Template Definition for Post IR/DR State for focus_tap and unfocus_tap Commands

If focus_tap or unfocus_tap requires different post IR/DR state from other commands producing both shift IR & DR, a separate SCOPE needs to be defined. The same format and rules for the SCOPE defined for set/compare commands can be applied here.

#### Parameterized template for SWCOMP support

SWCOMP is a special hardware feature in a TAP to compare the value of a selected TDR with pre-set high and low limit values and to capture the result of the comparison. In order to support SWCOMP feature in SPF, more fine-grained control of the shift DR operation is required because multiple fields of a selected TDR may need to be compared in SWCOMP logic. User can achieve the fine control of the shift DR operation by redefining the details of shift_dr operation and using parameterized template. This is shown below. More details of SWCOMP support can be obtained in “Pattern Framework Spec – Test” document.

##### SOC Tap template for SWCOMP

DEFAULT_TEMPLATE|TEMPLATE TAP <template name>

{

PARAMETERS = {

comma separated list of [$<var name> | @<array name>];

}

...

SCOPE : <set, compare, tap_raw_shift_irdr> = {

STEPS = {

shift_ir;

to_state = <state> ;

[pass|cycle] ;

shift_dr ;

to_state = <state> ;

[pass|cycle] ;

}

BASIC_OPERATION = {

shift_dr = {

shift_by <value> | <template variable>;

to_state = <state> ;

for ($<local var> = <initial value>; <final value>; <increment or decrement>)

{

shift_by <value> | <template variable>;

to_state = “<state>”;

}

...

}

}

}

}

##### Tap link template for SWCOMP

DEFAULT_TEMPLATE|TEMPLATE TAP <template name>

{

PARAMETERS = {

comma separated list of [$<var name> | @<array name>];

}

...

SCOPE : <set, compare, tap_raw_shift_irdr> = {

STEPS = {

link_ir;

to_state = <state> ;

[pass|cycle] ;

shift_ir;

to_state = <state> ;

[pass|cycle] ;

link_dr ;

to_state = <state> ;

[pass|cycle] ;

shift_dr ;

to_state = <state> ;

[pass|cycle] ;

}

BASIC_OPERATION = {

shift_dr = {

shift_by <value> | <template variable>;

to_state = <state> ;

for ($<local var> = <initial value>; <final value>; <increment or decrement>)

{

shift_by <value> | <template variable>;

to_state = “<state>”;

}

...

}

}

}

}

#### SOC TAP Template Example

DEFAULT_TEMPLATE TAP template_tap

{

scope : focus_tap, unfocus_tap = {

steps = {

shift_ir ;

to_state = “Post Sel IR State”;

pass itpp “pass string 1”;

shift_dr ;

to_state = “Post Sel DR State”;

pass itpp “pass string 2”;

cycle 50 ;

}

}

scope : set, compare, tap_raw_shift_irdr = {

steps = {

shift_ir ;

to_state = “Post IR State 1”;

pass itpp “pass string 3”;

shift_dr ;

to_state = “Post DR State 1”;

pass itpp “pass string 4”;

}

}

scope : execute, tap_raw_shift_ir = {

steps = {

shift_ir ;

to_state = “Post IR State 2”;

pass itpp “pass string 5”;

}

}

}

#### TAP Link Template Example

DEFAULT_TEMPLATE TAP template_tap

{

scope : set, compare, tap_raw_shift_irdr = {

steps = {

link_ir ;

to_state = “Post Link IR State”;

shift_ir ;

to_state = “Post IR State 1”;

pass itpp “pass string 3”;

link_dr ;

to_state = “Post Link DR State”;

shift_dr ;

to_state = “Post DR State 1”;

pass itpp “pass string 4”;

}

}

scope : execute, tap_raw_shift_ir = {

steps = {

link_ir ;

to_state = “Post Link IR State”;

shift_ir ;

to_state = “Post IR State 2”;

pass itpp “pass string 5”;

}

}

}

### STF Testport(TP) Templates and Transforms

SPF supports packet generation at the test port access interface.

- Mode 1: Packets generated at the STF interface are converted to TP packets.

- Mode 2: Direct TP packets are generated for TP resource accesses.

SPF requires user to provide a template file in order to generate packets at the Test port. Template file is provided to SPF through the following command line option –

--templateFile <template file name>

#### Mode1: STF->TP packet transformation

Short data mode:

In this mode SPF generates 1:1 TP packet to STF packet under the given condition that only limited number of bits in the STF data word are needed at the TP. In order to enable short data mode based packet transformation, the tool requires user to specify the test port architecture in the collateral file and provide the transformation techniques in a template file as well. Refer to the Test Port architecture section for details on the format for specifying Test Port in the collateral file. The STF architecture needs to be modified to include TP in the ring hierarchy depending on the way test port is configured in the network. No particular change is required in the test sequence file for packet transformation.

Example TP architecture definition in the collateral file – no changes to the STF architecture in this example as TP is configured outside of the STF ring:

TEST_PORT STF_Test_Port_Name {

//@DELAY = 3; //Delay is optional and only work if Test port defined

//in ring hierarchy

INTERFACE STF_TestPort_interface;

PACKETS {

TestPortIn TP_IN;

TestPortIn TP_OUT;

}

REGISTERS {

RegisterTpA {

@ADDRESS = 5'h1;

@SIZE    = 9;

@DEFAULT = 'd111;

ALIASES {

aliasA {

} [0:2];

aliasB {@DEFAULT = 'b10;}[3:4];

aliasC {}[5:6];

aliasD {

//@DEFAULT = 2'b10;

MNEMONICS { T1='b01, T2='b11, T3='b10, ALL='b11; }

}[7:8];

}

}

}

}

STF_ARCHITECTURE {

…

TOPOLOGY {

<Test_Port_Name>: <Controller>;

}

HIERARCHY {

<CTL|HUB Name> : <Ring inst list>;

}

…

}

INTERFACE {

STF_Port_Name :

STF_IN[31:0]  = stf_in[32:1],

STF_OUT[31:0] = stf_out[32:1];

STF_TestPort_interface :

TP_IN[32:0] = { tp_in[1],tp_in[2],tp_in[3],tp_in[4],tp_in[5],

tp_in[10],tp_in[6],tp_in[7],tp_in[8],tp_in[9],

tp_in[21],tp_in[22],tp_in[23],tp_in[24],tp_in[25],

tp_in[13],tp_in[12],tp_in[11],tp_in[31],tp_in[30],

tp_in[14],tp_in[15],tp_in[16],tp_in[17],tp_in[18],

tp_in[26],tp_in[27],tp_in[28],tp_in[29],tp_in[33],

tp_in[20],tp_in[19],tp_in[32]},

TP_OUT[32:0] = tp_out[34:2];

}

Example template file definition:

//Template file

DEFAULT_TRANSFORM TEST_PORT STF STF2TestPort

{

TP_IN:STF_IN = {

STF_CODE[2:0] = OPCODES[2:0];

GROUP = GID[2:0];

STF_DATA[19:0] = DATA[19:0];

CONTROL_BITS = 2'b10;

}

TP_OUT:STF_OUT = {

STF_CODE[1:3] = OPCODES[0:2];

GROUP[2:0] = GID[0:2];

STF_DATA[0:18] = DATA[2:20];

CONTROL_BITS = 'b10;

}

}

Example test sequence – no change in the test sequence specification:

//example.ts

set_stf_packet input 'd4 1 'b0100 1; //HUB1 is set to group1

cycle 24;

focus_stf HUB1;

set_stf_packet input 'd4 1 'b0100 1; //HUB1 is set to group1

focus_stf HUB1;

cycle 34;

@set tap_auto_mode on;

set ChildGroupMemberships->group_2 = 'b1;

set ChildGroupResponders->group_2 = 'b1;

set TrackerDelay[8] = 'b1;

set TrackerDelay->tracker_delay_value = 'd5;

set HubDebug1[4] = 'b0; //Error if commented

set HubDebug1[3] = 'b1; //Clock ratio Change

flush;

The above test sequence translates to the following itpp.

//example.itpp

rem: in: tp_in : STF_CODE = 100 GROUP = 001 STF_DATA = 00000000000000010100 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000010100001010000) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

vector: TapMode(0), 24 ;

rem: in: tp_in : STF_CODE = 100 GROUP = 001 STF_DATA = 00000000000000010100 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000010100001010000) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

vector: TapMode(0), 34 ;

rem: in: tp_in : STF_CODE = 110 GROUP = 001 STF_DATA = 00000000000000000011 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000000011001011000) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 111 GROUP = 001 STF_DATA = 00000000000000000111 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000000111001011100) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 110 GROUP = 001 STF_DATA = 00000000000000010100 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000010100001011000) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 111 GROUP = 001 STF_DATA = 00000000000000000111 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000000111001011100) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 110 GROUP = 001 STF_DATA = 00000000000000001000 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000001000001011000) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 111 GROUP = 001 STF_DATA = 00000000000100000101 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000100000101001011100) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 110 GROUP = 001 STF_DATA = 00000000000000011001 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000011001001011000) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 111 GROUP = 001 STF_DATA = 00000000000000001000 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(110000000000000000001000001011100) tp_out(LHXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

#### Mode 2: Direct TP packet generation

This mode enables direct Test Port resource access through the template file. The steps for packet construction should be defined in the template file under the DEFAULT_TEMPLATE procedure. The test port architecture needs to be specified in the collateral file.  The STF architecture needs to be modified to include TP in the STF ring hierarchy depending on the way test port is configured in the network. The test sequence needs to call focus_stf  command on the test port when needed to access TP resources.

Example collateral definition where TP is part of the ring:

TEST_PORT Test_Port_Name {

//@DELAY = 3; //Delay is optional and only work if Test port defined

//in ring hierarchy

INTERFACE STF_TestPort_interface;

PACKETS {

TestPortIn TP_IN;

TestPortIn TP_OUT;

}

REGISTERS {

RegisterTpA {

@ADDRESS = 5'h1;

@SIZE    = 9;

@DEFAULT = 'd111;

ALIASES {

aliasA {

} [0:2];

aliasB {@DEFAULT = 'b10;}[3:4];

aliasC {}[5:6];

aliasD {

//@DEFAULT = 2'b10;

MNEMONICS { T1='b01, T2='b11, T3='b10, ALL='b11; }

}[7:8];

}

}

RegisterTpB {

@ADDRESS = 5'h2;

@SIZE    = 7;

ALIASES {

aliasA {}[0:2];

aliasB {}[3:4];

aliasC {}[5:6];

}

}

}

}

STF_ARCHITECTURE {

…

TOPOLOGY {

<Test_Port_Name>: <Controller>;

}

HIERARCHY {

<Test_Port_Name > : CTL , <ring inst list>

…

}

…

}

Example template file definition:

//Template file

DEFAULT_TEMPLATE TEST_PORT  stf_test_port

{

Scope : Set,compare = {

TP_IN = {

REG_INDEX[0:4] = REGISTER->ADDRESS;

REG_DATA = REGISTER->VALUE;

CONTROL_BITS[0:1] = 2'd3;

}

TP_OUT = {

REG_INDEX[6:2] = REGISTER->ADdRESS;

REG_DATA[15:10] = ReGIsTER->VaLUE;

CONTROL_BITS = 2'd3;

}

}

}

DEFAULT_TRANSFORM TEST_PORT STF STF2TestPort

{

TP_IN:STF_IN = {

INPUT_VALID = INPUT_VALID;

OUTPUT_VALID = OUTPUT_VALID;

STF_CODE[2:0] = OPCODES[2:0];

GROUP = GID[2:0];

STF_DATA[19:0] = DATA[19:0];

CONTROL_BITS = 2'b10;

}

TP_OUT:STF_OUT = {

INPUT_VALID = INPUT_VALID;

OUTPUT_VALID = OUTPUT_VALID;

STF_CODE[1:3] = OPCODES[2:0];

GROUP[2:0] = GID[0:2];

STF_DATA[0:18] = DATA[2:20];

CONTROL_BITS = 'b10;

}

}

Example test sequence – no change in the test sequence specification:

//example.ts

focus_stf STF_Test_Port_Name;

compare RegisterTpA[0:2] = 'b01;

compare RegisterTpA->aliasB = 'b11;

flush;

The above test sequence translates to the following itpp.

//example.itpp

rem: in: tp_in : INPUT_VALID = 1 OUTPUT_VALID = 0 REG_INDEX = 00001 REG_DATA = 000000000 CONTROL_BITS = 10 STF_CODE = 10 REG = RegisterTpA TESTPORT = STF_Test_Port_Name

rem: out: tp_out : INPUT_VALID = 1 OUTPUT_VALID = 0 REG_INDEX = 00001 REG_DATA = X0111XXXX CONTROL_BITS = 10 REG = RegisterTpA TESTPORT = STF_Test_Port_Name

vector: tp_in(010000000000000000000000001010001) tp_out(HLXXXXXXXHHXXXXXXXXXXXXXXXLLLLHXH);

### STF to MCI Transform

The STF to MCI transform provides the capability to transform STF packets to a destination packet type like MCI. Since MCI is not a peripheral interface the MCI packets will utlitmately be transformed to another destination like DDR or TAP, which is determined by the selected MCI transform. SPF tool behavior is described below –

- It is mandatory to specify a transform to define STF to MCI conversion.

- It is mandatory to specify a transform to define MCI to a peripheral interface conversion.

- PACKETIZE function call should be specified in STEPS. This is called to transform STF packets generated from the sequence given in the .spf file to MCI packets. The PACKETIZE construct must provide mapping for every bit in the MCI packet.

- PRE_AMBLE and POST_AMBLE are not allowed. SPF will terminate with error if either is specified.

The following example presents a STF-to-MCI transform definition

///#################### MCI2STF #################

TRANSFORM MCI STF STF2MCI_2TO1_MODE {

// Mapping functions

MCI_IN:STF_IN = {

MCI_IN[19:0] = STF_IN[$1:$2];

VALID = STF_IN[$3];

}

MCI_OUT:STF_OUT = {

MCI_OUT[19:0] = STF_OUT[$1:$2];

VALID = STF_OUT[$3];

}

//Delay specification

DELAY = {

MCI : 2;

STF : 2;

}

// Packet conversion steps

STEPS = {

PACKETIZE = {

MCI_IN:STF_IN   (21, 2, 0);

MCI_OUT:STF_OUT (21, 2, 1);

}

PACKETIZE = {

MCI_IN:STF_IN   (41, 22, 0);

MCI_OUT:STF_OUT (41, 22, 1);

}

}

}

### MCI Transform

SPF requires user to provide a template file in order to transform MCI packets to intended destination test access type. Template file is provided to SPF through the following command line option –

--templateFile <template file name>

SPF now supports MCI parallel mode(MCIToDDR transformation) and serial mode(MCI-to-TAP transformation).

#### MCI to DDR Transform

The MCI transform provides the capability to transform MCI packets to the destination packet type like DDR. SPF tool behavior is described below –

- It is mandatory to specify a transform to define MCI to DDR conversion.

- If INTERFACE_REDEFINITION section is not specified in the transform, the INTERFACE defined in the collateral will be used.

  - The INTERFACE_REDEFINITION section can map the packets defined in the collateral to different pins.

  - INTERFACE_REDEFINITION section can change the type associated with an interface from input to output or output to input.

  - INTERFACE_REDEFINITION can map a different packet not specified in the INTERFACE definition in the collateral, however specified in the PACKETS section of DDR section.

  - Bit swizzling can be specified in the INTERFACE_REDEFINITION in the transform.

  - REUSE_GROUP keyword can be used to re-use the pin group mapping provided in the collateral so that the pin group mapping need not be specified again if it doesn’t change from the collateral.

- It is mandatory to specify mci_bubble_insertion attribute in the transform which defines the number of bubbles inserted for every n number of mci packets and also the bubble value. SPF uses this attribute information to construct the DDR packets.

- There can only be one PACKETIZE function call in STEPS. This is called iteratively to transform mci packets provided by the user in the .spf sequence to DDR packets based on the mci_bubble_insertion attribute. Note that in the PACKETIZE construct, mapping for every interface defined in the INTERFACE/ INTERFACE_REDEFINITION section should exist except the clock. PACKETIZE can only be specified in STEPS and POST_AMBLE

- ‘ INTERFACE_TRANSACT_COUNT is a recognized SPF tool variable that counts the number of vectors produced for the destination test access type. It is reset at the end of every mci burst.

The following are example templates for some of the modes for MCI.

- Two channel mode:

TRANSFORM DDR MCI MCI2DDR_2IN {

// Packet to interface mapping can be redefined

INTERFACE_REDEFINITION {

DDR_interface :

USER_DEFINED = {

INPUT = {

DDR0[9:0]  = REUSE_PINGROUP,// pin group from collateral will be used for DDR0.

DDR1[9:0] = ddr1[9:0],

DQS[7:0] = dqs[7:0];

}

}

}

//Clock specification

CLOCK = {

DQS: 8‘b01, 8‘b0 ;

}

// Attribute specification that defines the transformation process

ATTRIBUTES = {

@set_attribute mci_bubble_insertion mci_packet_count = 3, mci_bubble_count = 3, mci_bubble_value = 3’b0;

}

// Transform procedure can be parameterized such as regular bubble insertion

PARAMETERS = {

$BUB_CNT;

}

CONST_VAR = {

$NOP_VAL = 3’b0;

}

DDR0:MCI_IN {

MCI_0 = MCI_IN{$1};

MCI_1 = MCI_IN{$2};

MCI_2 = MCI_IN{$3};

VALID = $4;

}

DDR1:MCI_IN {

MCI_0 = MCI_IN{$1};

MCI_1 = MCI_IN{$2};

MCI_2 = MCI_IN{$3};

VALID = $4;

}

DDR0:- {

MCI_0 = $NOP_VAL;

MCI_1 = $NOP_VAL

MCI_2 = $NOP_VAL;

VALID = $4;

}

DDR1:- {

MCI_0 = $NOP_VAL;

MCI_1 = $NOP_VAL;

MCI_2 = $NOP_VAL;

VALID = $4;

}

PRE_AMBLE = {

comment rem “Two channel input only mode”;

}

STEPS =  {

PACKETIZE = {

DDR0:MCI_IN (0, 1, 2, 1);

DDR1:MCI_IN(3, 4, 5, 1);

}

}

POST_AMBLE  = {

LOCAL_VAR = {

$iter = 0;

}

// Addition of bubble packets at end of seq

$iter = ($BUB_CNT%6 == 0) ? $BUB_CNT/6:$BUB_CNT/6+1;

for($i=1; $iter; 1) {

PACKETIZE = {

DDR0:- (1);

DDR1:- (1);

}

}

// Additional pkts to make mod 2 interface transactions

$iter = (‘INTERFACE_TRANSACT_COUNT%2 == 0)? 2:1;

for($i=$iter; 1; 1) {

PACKETIZE = {

DDR0:- (1);

DDR1:- (1);

}

}

@set clock_toggle DQS off;

//Addition of DDR nop at end of seq

PACKETIZE = {

DDR0:- (0);

DDR1:- (0);

}

}

- Input output mode

TRANSFORM DDR MCI MCI2DDR_INOUT {

// Packet to interface mapping can be redefined

INTERFACE_REDEFINITION {

DDR_interface :

USER_DEFINED = {

INPUT = {

DRR0[9:0]  = ddr0[9:0],

DQS[7:0] = dqs[7:0];

}

OUTPUT = {

DRR1[9:0]  = ddr1[9:0];

}

}

}

//Clock specification

CLOCK = {

DQS: 8‘b01, 8‘b0 ;

}

//Delay specification

DELAY = {

MCI : 5;

DDR : 1;

}

// Attribute specification that define the transformation process

ATTRIBUTES = {

@set_attribute mci_bubble_insertion mci_packet_count = 3, mci_bubble_count = 3, mci_bubble_value = 3’b0;

}

// Transform procedure can be parameterized such as regular bubble insertion

PARAMETERS = {

$BUB_CNT;

}

CONST_VAR = {

$NOP_VAL = 3’b0; $NOP_OUT = ‘bXXX;

}

DDR0:MCI_IN {

MCI_0 = MCI_IN{$1};

MCI_1 = MCI_IN{$2};

MCI_2 = MCI_IN{$3};

VALID = $4;

}

DDR1:MCI_OUT {

MCI_0 = MCI_OUT{$1};

MCI_1 = MCI_OUT{$2};

MCI_2 = MCI_OUT{$3};

}

DDR0:- {

MCI_0 = $NOP_VAL;

MCI_1 = $NOP_VAL

MCI_2 = $NOP_VAL;

VALID = $4;

}

DDR1:- {

MCI_0 = $NOP_OUT;

MCI_1 = $NOP_OUT;

MCI_2 = $NOP_OUT;

}

STEPS =  {

PACKETIZE = {

DDR0:MCI_IN(0, 1,  2, 1);

DDR1:MCI_OUT(0, 1, 2);

}

}

PRE_AMBLE = {

comment rem “Input output mode”;

}

POST_AMBLE  = {

LOCAL_VAR = {

$iter = 0; $CLK = 0;

}

// Addition of bubble packets at end of seq

$iter = ($BUB_CNT%3 == 0) ? $BUB_CNT/3 : $BUB_CNT/3+1;

for($i =1; $iter; 1) {

PACKETIZE= {

DDR0:- (1);

DDR1:- ();

}

}

// Additional pkts to make mod 2 interface transactions

$iter = (‘INTERFACE_TRANSACT_COUNT%2 == 0)? 2:1;

for($i=$iter; 1; 1) {

PACKETIZE= {

DDR0:- (1);

DDR1:- ();

}

}

@set clock_toggle DQS off;

//Addition of DDR nop at end of seq

PACKETIZE= {

DDR0:- (0);

DDR1:- ();

}

}

#### MCI to TAP Transform

The MCI-to-TAP transform provides the capability to drive MCI content through TAP. The input raw MCI packets will be transformed to generate ITPP at the TAP interface. SPF tool behavior is described below –

- The MCI input/output mode is not supported.

- No special handling is provided unlike parallel(MCIToDDR) mode requirements:

      - No periodic bubble insertion.

      - No padding for full DDR

      - No even number of DDR packets

- MCI-to-TAP transformation is achieved through transform provided in the template file. The transform provides a mechanism to perform initial configuration settings through the procedure specified in the PREAMBLE and to transform the raw mci packet stream to tap transactions through the procedure defined in the STEPS. An optional POSTAMBLE can be provided, that will be executed at the end of the mci packet stream marked by unfocus_mci in the spf sequence.

- The following tap commands can be specified in the PRE_AMBLE, STEPS, POSTAMBLE to defined the transformation - focus_tap, set, compare and flush.

- There can be more than one flush block consuming MCI packet in STEPS.

- If the procedural blocks PRE_AMBLE, STEPS and POST_AMBLE contain any flush block commands like set, compare, tap_raw_shift, execute, they should be terminated with a flush in the procedural block. There may be an optional focus_tap prior to the beginning of a flush block in the procedural block.

- MCI and TAP collaterals should be provided in the command line options.

- MCI-to-TAP transformation is supported for both SOC and tap link type of network.

- Current tool limitation: IR optimization is supported only for SOC type of tap network. The directive for IR optimization should be set in the test sequence prior to the beginning of the raw MCI packet sequence.

Label Handling

If labels are associated with any of the packet in MCI test sequence, the labels are printed in itpp along with the first TAP transaction consuming MCI packet in STEPS. In case of a loop, if a loop statement consumes MCI packet, the label is attached only with the first TAP transaction in the loop. An example is illustrated in the Pattern Framework Spec-Test document.

The following is an example transform for MCI-to-TAP transformation:

TRANSFORM TAP MCI TAP2MCI {

GLOBAL_VAR = {

$toggle = 'b0;

}

PRE_AMBLE = {

focus_tap IMPH;

set IMPH_MCICFG->RING_MCI_RESET = 'b1;

set IMPH_MCICFG->SA_PDI_MCISERIAL_ENABLE = 'b10;

set IMPH_MCICFG->SA_PBIST_TARGET_EN = 'b1;

flush;

}

STEPS = {

$toggle= ~$toggle;

focus_tap IMPH;

set IMPH_MCISERIAL->TX_ID = $toggle;

set IMPH_MCISERIAL->DATA = PACKET::MCI[19:0];

set IMPH_MCISERIAL->MODE = 'b1;

set IMPH_MCISERIAL->DATA_VALID =  PACKET::MCI[20];

set IMPH_MCISERIAL->TX_VALID = 'b1;

flush;

}

POST_AMBLE = {

}

}

### CR transform

For CR content transformation, the user is required to provide a transform_seq which defines the transport protocol to specifiy the required steps to transform a CR sequence to the required test access mechanism. This provides a description of the transport mechanism in the design to access a register space. The transform contains tap/stf sequence over which cr data is sent. The sequence may contain the tap/stf endpoint name, register information, etc. or the information can be parameterized. A CR transform is to be defined for each CR type. If there are multiple register register spaces for a given CR type, the taps/stf endpoints and registers to be used can be parameterized. When the user specifies the CR transaction intent in the spf sequence file, the transform is eventually used to convert the CR register transactions to destination TAM transactions. The tap/stf sequence specifies the tap/stf endpoint through which the CR data is to be sent. The sequence uses information from the CRIF file for register file, register, field related information apart from the actual value to be set/compared for the register which is provided as set/compare transaction in the spf sequence.

Please refer to TRANSFORM_SEQ section for syntax details on writing a transform sequence.

#### Crif property variables

For each register space of interest, CR properties which are not part of the predefined ones need to be defined and their values need to be extracted from specific attributes in CRIF files, they should be defined in ‘cr_spec’ section. Please refer to CR Collateral Definition under Project Level Specification section for further details on collateral information extraction for CRs. The value for the CR properties may need to be used in the transform sequence. There are three ways of specifying crif property variables at each level described in the table below:

| Property | CR scope | Variable syntax in transform | Value type |
| --- | --- | --- | --- |
| Register file level | Register File | REG_FILE::<property name> | type defined in cr_spec section |
| Register level | Register | REG::<property name> | type defined in cr_spec section |

#### Tool recognized variables

For every register space in every CR fabric type, the following CR properties are predefined in tool and collected from CRIF by reading the corresponding attributesand can be referenced in the transform using a tick(‘) prepended to the variable name as given below. Please refer to Pattern Framework Spec – Design Collateral Spec for CR collateral information and Project level Specification section in this document(Pattern Framework Spec – Template) for details on CR collateral information extraction from CRIF files.

| Property | CR scope | Attribute name | Variable syntax in transform | Value type |
| --- | --- | --- | --- | --- |
| Register file name | Register File | name | ‘REG_FILE_NAME | string |
| Register name | Register | name | ‘REG_NAME | string |
| Register size |  | size | ‘REG_SIZE | verilog_value |
| Register field name | Field | name | ‘FIELD_NAME | string |
| Field access type |  | access | ‘ACCESS | string |
| Field description |  | description | ‘FIELD_DESC | string |
| Begin bit index for field |  | bitOffset | ‘FIELD_BEGIN_INDEX | verilog_value |
| Field width |  | bitWidth | ‘FIELD_WIDTH | verilog_value |
| RTL signal name for field |  | ValRTLSignal | ‘RTL_SIGNAL | string |
| Reset value |  | reset | ’RESET_VALUE | verilog_value |
| Async reset signal |  | ResetSignal | ‘ASYNC_RST_SIGNAL | string |
| Sync reset signal |  | SyncResetSignal | ‘SYNC_RST_SIGNAL | string |

A property name is assigned to each property with a reserved keyword so that the keyword can be used to reference the property in user interaction. During CR sequence transformation, the value of the attribute corresponding to a property may be obtained using the property name keyword.

There are additional variables maintained in the tool internally to pass on the information regarding the CR register set/compare transactions. Following are the variables listed –

| Attribute | Description | Predefined property name | Value type |
| --- | --- | --- | --- |
| CR set/compare value | Value to be assigned or strobed from a CR register specified through set/compare command in a flush block | ‘DATA | verilog_value |
| First byte enable | By default is set to ‘hF | ‘FBE | verilog value |
| Second byte enable | If size of register is 64, second byte enable is set to ‘hF in addition to | ‘SBE | verilog value |

Addional set of properties are predefined for a specific fabric type such as IOSF-SB as described in the following subsection.

##### IOSF-SB

The following table shows the properties pre-defined for IOSF-SB fabric type.

| Property | CR scope | Attribute name | Predefined property name | Value type |
| --- | --- | --- | --- | --- |
| IOSF-SB Port ID | Register File | portid | ‘PORTID | verilog_value |
| Function ID |  | SB_Fid | ‘SB_FID | verilog_value |
| Base Address |  | SB_Bar | ‘SB_BAR | verilog_value |
| Opcode |  | opcode | ‘OPCODE | enum |
| Fabric type |  | Fabric | ‘FABRIC | string |
| Address offset | Register | addressOffset | ‘ADDR_OFFSET | verilog_value |

##### CRI

The following table shows the properties pre-defined for CRI fabric type.

| Property | CR scope | Attribute name | Predefined property name | Value type |
| --- | --- | --- | --- | --- |
| Address offset | Register | addressOffset | ‘ADDR_OFFSET | verilog_value |

SPF supports CR transactions over TAP or STF network. There are two types of CR transforms CR to TAP and CR to STF supported and are discussed below.

#### CR to TAP

TRANSFORM definition:

TRANSFORM_SEQ|DEFAULT_TRANSFORM_SEQ TAP CR::<CR type> <func name> {

PARAMETERS = {

comma separated list of [$<var name>[{type=[string|integer|verilog_value}] | @<array name>];

}

CONST_VAR = {

$<var name> = <value>;

}

GLOBAL_VAR = {

$<var name> = <value>;

}

SCOPE : <list of commands> = {

STEPS = {

<local variable definition>

focus_tap $tap;

set $reg = ‘SET_DATA;

…… ;

}

}  }

<local var definition>::=

LOCAL_VAR = {

<var> = <initial value>;

}

<Sequential steps>::=<Stmt> | <for loop>

<for loop>::=

for ($<loop variable>=<initial value>; <final value>; <increment or decrement>) {

<Stmt>;

...

}

<Stmt>::= [<assignment> |<commands>|]

<assignment>::= [<simple assignment> | <arithmetic assignment> | <conditional assignment>]

<commands>::= comment, pass itpp, focus_tap, focus_tap_rf, set, compare, execute, tap_raw_shift, flush, cycle

<src spec> ::= <src field>

The following spec below describes a CR to TAP transformation:

Example 1:

TRANSFORM_SEQ TAP CR::IOSF_SB TAP2IOSF {

PARAMETERS = {

$TAP{ type = string;}, $NP_WRITE, $NP_OVRD_FLAG;

}

SCOPE : set = {

STEPS = {

LOCAL_VAR = { $addr_width = 0; }

$addr_width = 'ADDR_WIDTH - 1;

focus_tap $TAP;

// 64-bit register transaction

if('SBE == 'b1111)

{

set SBEXTDATAWR = 'SET_DATA[63:32];

flush;

}

set SBMSGGO[63:48] = 'ADDR_OFFSET;

set SBMSGGO[29:27] = 'SB_BAR;

// ext addrlen

if('ADDR_OFFSET[$addr_width:16] != 'b0)

{

set SBMSGGO[30:30] = 'b1;

set SBEXTADDR->extAddr = 'ADDR_OFFSET[$addr_width:16];

}

else

{

set SBMSGGO[30:30] = 'b0;

}

set SBMSGGO[15:8] = 'PORTID;

set SBMSGGO[47:40] = 'SB_FID;

set SBMSGGO[23:16] = 'OPCODE_WRITE;

set SBMSGGO->msgtrigsel = 'b00;

//NP_WRITE

if($NP_OVRD_FLAG)

{

set SBMSGGO->nptrans = $NP_WRITE;

}

else

{

if('OPCODE == "MEM-SB" || 'OPCODE == "IO-SB" || 'OPCODE == "CFG-SB")

{

set SBMSGGO->nptrans = 'b1;

}

else

{

set SBMSGGO->nptrans = 'b0;

}

}

// 64-bit register transaction

if('SBE == 'b1111)

{

set SBMSGGO->data64en = 'b1;

}

else // 32-bit register transaction

{

set SBMSGGO->data64en = 'b0;

}

set SBMSGGO[35:32] = 'FBE;

set SBMSGGO[39:36] = 'SBE;

set SBMSGGO[26:24] = 'b000;

set SBMSGGO[95:64] = 'SET_DATA[31:0];

flush;

}

}

SCOPE: compare = {

STEPS = {

LOCAL_VAR = { $addr_width = 0; }

$addr_width = 'ADDR_WIDTH - 1;

focus_tap $TAP;

set SBMSGGO[63:48] = 'ADDR_OFFSET;

set SBMSGGO[29:27] = 'SB_BAR;

// EXT ADDR

if('ADDR_OFFSET[$addr_width:16] != 'b0)

{

set SBMSGGO[30:30] = 'b1;

set SBEXTADDR->extAddr = 'ADDR_OFFSET[$addr_width:16];

}

else

{

set SBMSGGO[30:30] = 'b0;

}

set SBMSGGO[15:8]  = 'PORTID;

set SBMSGGO[47:40] = 'SB_FID;

set SBMSGGO[23:16] = 'OPCODE_READ;

set SBMSGGO->msgtrigsel = 'b00;

// NP write

if($NP_OVRD_FLAG)

{

set SBMSGGO->nptrans = $NP_WRITE;

}

else

{

set SBMSGGO->nptrans = 'b1;

}

// 64-bit register transaction

if('SBE == 'b1111)

{

set SBMSGGO->data64en = 'b1;

}

else // 32-bit register transaction

{

set SBMSGGO->data64en = 'b0;

}

set SBMSGGO[35:32] = 'FBE;

set SBMSGGO[39:36] = 'SBE;

set SBMSGGO[26:24] = 'b000;

flush;

cycle 10;

// 64-bit register transaction

if('SBE == 'b1111)

{

compare SBMSGRSP[95:32] = 'COMPARE_DATA;

}

else // 32-bit register transaction

{

compare SBMSGRSP[63:32] = 'COMPARE_DATA;

}

compare SBMSGRSP->status = 'b11;

flush;

}

}

}

In the above transform_seq definition, the mechanism to transform IOSF_SB register transactions to tap sequence is given. The PARAMETERS section defines variables that need to be provided by the user through the test sequence, please refer to Pattern Framework Spec – Test for details. The parameter variables are used in the transform body – eg: in the scopes for set and compare commands. It is to be noted that pre-defined tool variables are references with tick(‘) preceding the property requested AND other crif properties like register level description with REG::DESC, where DESC is assumed to be specified as a property in the cr_spec. Please refer to Pattern Framework Spec – Test for details on usage of the template.

#### CR to STF

TRANSFORM definition:

TRANSFORM_SEQ|DEFAULT_TRANSFORM_SEQ STF CR::<CR type> <func name> {

PARAMETERS = {

comma separated list of [$<var name>[{type=[string|integer|verilog_value}] | @<array name>];

}

CONST_VAR = {

$<var name> = <value>;

}

GLOBAL_VAR = {

$<var name> = <value>;

}

SCOPE : <list of commands> = {

STEPS = {

<local variable definition>

focus_tap $tap;

set $reg = ‘DATA;

…… ;

}

}  }

<local var definition>::=

LOCAL_VAR = {

<var> = <initial value>;

}

<Sequential steps>::=<Stmt> | <for loop>

<for loop>::=

for ($<loop variable>=<initial value>; <final value>; <increment or decrement>) {

<Stmt>;

...

}

<Stmt>::= [<assignment> |<commands>|]

<assignment>::= [<simple assignment> | <arithmetic assignment> | <conditional assignment>]

<commands>::= focus_stf, set, compare, flush, cycle, set_stf_packet

<src spec> ::= <src field>

The following spec below describes a CR to TAP transformation:

Example 1:

TRANSFORM_SEQ STF CR::IOSF_SB IOSF2STF {

PARAMETERS = {

$STF_EP{type = string;}, $TAP{type = string;}, $BCAST, $NP_WRITE, $NP_OVRD_FLAG, $repeat;

}

SCOPE : set = {

STEPS = {

if('SB_FID > 'b0)

{

focus_tap $TAP;

set  POVRSBFID->pmsgactive = 1'b1;

set  POVRSBFID->pmsgfid = 'SB_FID;

flush;

}

focus_stf $STF_EP;

set STF_PMBIN_REG->addr = 'ADDR_OFFSET[15:0];

if('ADDR_OFFSET[47:16] != 'b0)

{

set STF_PMBIN_REG->ext_addr = 'b1;

set STF_PMBIN_REG->addr_hi = 'ADDR_OFFSET[47:16];

}

else

{

set STF_PMBIN_REG->ext_addr = 'b0;

}

set STF_PMBIN_REG->bar = 'SB_BAR;

set STF_PMBIN_REG->dest = 'PORTID;

set STF_PMBIN_REG->opcode = 'OPCODE_WRITE;

set STF_PMBIN_REG->broadcast_en = $BCAST;

if($NP_OVRD_FLAG)

{

set STF_PMBIN_REG->np_write = $NP_WRITE;

}

else

{

if('OPCODE == "MEM-SB" || 'OPCODE == "IO-SB" || 'OPCODE == "CFG-SB")

{

set STF_PMBIN_REG->np_write = 'b1;

}

else

{

set STF_PMBIN_REG->np_write = 'b0;

}

}

set STF_PMBIN_REG->be = 'FBE;

set STF_PMBIN_REG->tag = 'b000;

set STF_PMBIN_REG->data = 'SET_DATA;

flush;

set_stf_packet input NOP 1 'h0 (repeat=$repeat);

}

}

SCOPE: compare = {

STEPS = {

if('SB_FID > 'b0)

{

focus_tap $TAP;

set  POVRSBFID->pmsgactive = 1'b1;

set  POVRSBFID->pmsgfid = 'SB_FID;

flush;

}

focus_stf $STF_EP;

set STF_PMBIN_REG->addr = 'ADDR_OFFSET[15:0];

if('ADDR_OFFSET[47:16] != 'b0)

{

set STF_PMBIN_REG->ext_addr = 'b1;

set STF_PMBIN_REG->addr_hi = 'ADDR_OFFSET[47:16];

}

else

{

set STF_PMBIN_REG->ext_addr = 'b0;

}

set STF_PMBIN_REG->bar = 'SB_BAR;

set STF_PMBIN_REG->dest = 'PORTID;

set STF_PMBIN_REG->opcode = 'OPCODE_READ;

if($NP_OVRD_FLAG)

{

set STF_PMBIN_REG->np_write = $NP_WRITE;

}

else

{

set STF_PMBIN_REG->np_write = 'b1;

}

set STF_PMBIN_REG->be = 'FBE;

set STF_PMBIN_REG->tag = 'b000;

flush;

set_stf_packet input NOP 1 'h0 (repeat=$repeat);

# Re-focus after a raw packet

focus_stf $STF_EP;

compare STF_PMBOUT_REG->data = 'COMPARE_DATA;

flush;

}

}

}

In the above transform_seq definition, the mechanism to transform IOSF_SB register transactions to stf sequence is given. The PARAMETERS section defines variables that need to be provided by the user through the test sequence, please refer to Pattern Framework Spec – Test for details. The parameter variables are used in the transform body – eg: in the scopes for set and compare commands. It is to be noted that pre-defined tool variables are references with tick(‘) preceding the property requested AND other crif properties like register level description with REG::DESC, where DESC is assumed to be specified in the cr_spec. Please refer to Pattern Framework Spec – Test for details on usage of the template.

## Project Level Specification

Project level specifications are provided in the following format.

project_spec {

<Project level specification>

...

}

### TAP Network Definition

If multiple TAP networks are available for a given SoC or multi-chip package and it is desired to handle those networks concurrently in the same test sequence (please refer to “Concurrent Testing with Multiple JTAG Ports” section in “Pattern Framework Spec - Test” document), it is required to specify full list of TAP networks as a project level specification using the following syntax.

tap_network_list {

<TAP network name>;

...

}

An example is given below.

tap_network_list {

Prim_network;

Sec_network;

Ter_network;

}

### TAP Spec Augmentation

If TAP spec has missing or incorrect TAP collateral information, it may be overridden or supplemented with the additional TAP collateral information provided in “TAP_SPEC_AUGMENT” section under “project_spec”. Currently, the override of TAP mode select configuration for SoC TAP network and the addition of individual TAP specific information may be provided in “TAP_SPEC_AUGMENT” section.

The syntax of “TAP_SPEC_AUGMENT” is defined as follows.

project_spec {

...

TAP_SPEC_AUGMENT {

SELECT_OVERRIDE {

<TAP> {

...

}

...

}

TAP_AUGMENT {

<TAP> {

...

}

. . .

}

}

}

#### TAP Mode Select Override

The TAP mode select information for a TAP may be redefined in “SELECT_OVERRIDE” subsection under “TAP_SPEC_AUGMENT”. The TAP mode select information provided for a TAP in “SELECT_OVERRIDE” section will override the information provided in TAP spec file. Similar to ‘SELECT’ section defined in TAP spec, ISOLATE, NORMAL and SHADOW modes may be defined for a TAP in this section. While SHADOW mode may be given optionally, it is mandatory to specify ISOLATE and NORMAL mode for the given TAP.

The syntax for “SELECT_OVERRIDE” section is defined as follows.

SELECT_OVERRIDE {

<TAP> {

NORMAL {

<TAP>=><register/field> = <value>;

...

}

ISOLATE {

<TAP>=><register/field> = <value>;

<TAP>=><register/field> = <value>; | DISABLED;

...

}

SHADOW {

<TAP>=><register/field> = <value>;

...

}

}

...

}

Each TAP mode may be specified with configurations of multiple TAP registers. In this case, in order to to set the TAP in the given mode, a sequence of TAP registers transactions are required and the order of the registers transactions sequence should be kept the same as that of the TAP registers specifications given in the mode definition.

In ISOLATE mode, DISABLED keyword may be specified after a certain register configuration to indicate that the given TAP is out of the active scan chain right after the given register configuration. If DISABLED keyword is not explicitly given in ISOLATE mode definition, the given TAP will be out of the active scan chain after the configuration of the last register specified in ISOLATE mode.

An example of “SELECT_OVERRIDE” section to override the select mechanism of a TAP is given below.

SELECT_OVERRIDE {

CNV_WIFI_APMG_TAP {

NORMAL {

CNVI_TOP_TAP=>reg_jtag_reset[2] = ‘b1;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = ‘b10;

CNVI_TOP_TAP=>reg_jtag_tap_bypass[3] = ‘b0;

}

ISOLATE {

CNVI_TOP_TAP=>reg_jtag_tap_bypass[3] = ‘b1;

DISABLED;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = ‘b00;

CNVI_TOP_TAP=>reg_jtag_reset[2] = ‘b0;

}

}

}

#### TAP Augment

Additional information specific to a TAP, which is missing in TAP spec file, may be specified in “TAP_AUGMENT” section. Currently, indirect registers definition and their access information may be provided in this section. Indirect registers are defined under “INDIRECT_REGISTERS” subsection and the SPF command sequences for writing and reading the indirect registers are defined under “INDIRECTION_COMMANDS” subsection.

The syntax of the TAP_AUGMENT section is given below.

TAP_AUGMENT {

<TAP> {

INDIRECT_REGISTERS {

<indirect register definitions>

}

INDIRECTION_COMMANDS {

<SPF sequences for writing and reading indirect registers>

}

}

...

}

The definition of an indirect register is similar to that of a static register in TAP spec file. In case of an indirect register definition ‘ADDRESS’ attribute should be present. The syntax for an indirect register definition is given below.

<register> {

@SIZE = <int>;

@ADDRESS = <value>;

@DEFAULT = <value>; // opt

ALIASES { // opt

<alias list>;

}

}

The SPF sequence required for writing an indirect register is defined in ‘WRITE’ subsection under “INDIRECTION_COMMANDS” and the sequence required for reading an indirect register is defined in ‘READ’ subsection. The syntax for “INDIRECTION_COMMANDS” section is given below.

INDIRECTION_COMMANDS {

WRITE {

DEFAULT {

<SPF sequence>

}

}

READ {

DEFAULT {

<SPF sequence>

}

}

}

<SPF sequence> = <set command> | <compare command> | <flush command>;

An example of TAP_AUGMENT section for indirect register definition is given below.

TAP_AUGMENT {

CNVI_TOP_TAP {

INDIRECT_REGISTERS {

reg_jtag_tck_cg_en {

@SIZE = 2;

@ADDRESS = ‘h80000008;

@default = ‘b00;

}

reg_jtag_reset {

@SIZE = 6;

@ADDRESS = ‘h80000004;

@default = ‘b000000;

}

reg_jtag_tap_bypass {

@SIZE = 14;

@ADDRESS = ‘h80000000;

@default = ‘h3ffe;

}

}

INDIRECTION_COMMANDS {

WRITE {

DEFAULT {

set ADDRESS = $ADDRESS; flush;

set DATA = $DATA; flush;

set TRANSCMD = ‘h01; flush;

}

}

READ {

DEFAULT {

set ADDRESS = $ADDRESS; flush;

set TRANSCMD = ‘h05; flush;

compare DATA = $DATA; flush;

}

}

}

}

...

}

### Fullchip Pin Definition

Fullchip pins which are not part of interface pins for test access mechanism supported by SPF - TAP, STF, MCI/DDR - need to be defined in interface_pin_def section of project_spec if direct pin transactions on those pins using pin_transact or delay commands are needed in SPF. A fullchip pin must be classified as one of INPUT, OUTPUT, INOUT and CLOCK pins. In each pin definition, the logical name and the physical name of the pin should be defined. The physical name should be a legitimate design name. The logical name should be a generic name reflecting the functionality of the pin. In test sequence a logical name is supposed to be used and the corresponding physical name is reported in ITPP. By defining a logical name different from the physical name and using the logical name in test sequence, test may become portable without change across products.

The syntax of fullchip pin definition in project_spec is given below.

project_spec {

interface_pin_def { //define external signal

INPUT = {

<logical pin name> = <physical pin name>;

...

}

OUTPUT = {

<logical pin name> = <physical pin name>;

...

}

INOUT  = {

<logical pin name> = <physical pin name>;

...

}

CLOCK  = {

<logical pin name> = <physical pin name>;

...

}

}

}

<physical pin name>::= <single pin spec>|{<single pin spec>, <single pin spec>, ...}

<single pin spec>::= <pin name>|<pin group name>[<size>-1:0]

An example of fullchip pin definition in the template file is shown below

project_spec {

interface_pin_def {

INPUT = {

input_logic_pin = dcv_chv_xxx;

inext_pin = bxx_extest;

}

OUTPUT = {

array_pin_group = {pgrid_array_1, xx_array[10:0],

xx_abc};

}

INOUT  = {

inout_ext_pin = xxbcs_fullchip;

}

CLOCK = {

bclock = core_drive_clock;

}

}

}

Tool restrictions :

      - Only logical names can be used in the sequence.

      - Logical pin names must be unique across all the pin definitions for INPUT, OUTPUT, INOUT and CLOCK.

      - Duplicate definitions for a physical pin is not allowed. In other words, two different logical pin names cannot map to the same physical pin.

      - Logical pin names and physical pin names cannot overlap with the reserved pin or pin group names for supported TAMs, such as TCK, TMS, TDI, TDO, TRST, stf_in, stf_out, tp_in, tp_out.

      - Physical pin names cannot overlap with the physical pin names corresponding to the reserved pin or pin group names such as TCK, TMS, TDI, TDO, TRST, stf_in, stf_out, tp_in, tp_out.

      - Physical pin names cannot overlap with the interface pins defined for TAMs given in template/transform.

      - Physical pin names cannot overlap with any of the NOA pins defined in the current sequence.

      - Only pins defined as INPUT or OUPUT or INOUT can be used in the pin_transact command.

      - Only pins defined as CLOCK can be used in the delay command.

### CR Collateral Definition

It is necessary to extract certain collateral information from CRIF file selectively because only a small subset of CR attributes in CRIF are required for tool internal process. Basic CR attributes common to every register space will be extracted from CRIF file by default by SPF. But it is required to get user input on parse directions for additional attributes because it is not always possible to know all the CR attributes required for CR sequence transformation and also it is possible that certain CR properties may be given with different attributes for different products. The user input on CR collateral parse directions should be defined in ‘cr_spec’ under ‘project_spec’ section.

#### CRIF File Consistency Check

CR CRIF files provided by user are expected follow the basic hierarchical structure as described in “Control Register Spec” in “Pattern Framework Spec - Design Collateral Spec” document. Register files are defined in top level, registers are defined under a register file, and register fields are defined under a register.

Any departure from the above structure will be identified by tool, and by default, a warning will be reported while such an inconsistency is ignored. User can override this tool behavior using the tool option ‘cr_error_on_inconsistent _crif’ described in “Tool Option” section. If this option is turned on, tool will produce an error for an inconsistency found in a given CRIF file.

A CR CRIF file example following the standard format is given below.

<registerFile>

<name>cnxsp_top/ddrphy_top[1]/ddrdata1_type</name>

<type>MSG</type>

...

<register>

<name>data_dll_status</name>

<size>8<\size>

...

<field>

<name>adc_done</name>

<ValRTLSignal>i_ddrdatadigtop.i_ddrdatacr</ValRTLSignal>

...

</field>

</register>

</registerFile>

The following examples show a few instances of inconsistencies in CRIF fomat.

Example 1: field appears outside the scope of a register

<registerFile>

<name>cnxsp_top/ddrphy_top[1]/ddrdata1_type</name>

<type>MSG</type>

...

<register>

<name>data_dll_status</name>

<size>8<\size>

...

<field>

<name>adc_done</name>

<ValRTLSignal>i_ddrdatadigtop.i_ddrdatacr</ValRTLSignal>

...

</field>

</register>

<field> <- Inconsistent structure

<name>adc_clock</name>

<ValRTLSignal>i_ddrdatadigtop.i_ddrdatacr_[1]</ValRTLSignal>

...

</field>

</registerFile>

Similarly, a ‘register’ appearing outside the scope of a ‘registerFile’ is also treated as an inconsistency.

Example 2: field appears in the scope of another field

<registerFile>

<name>cnxsp_top/ddrphy_top[1]/ddrdata1_type</name>

<type>MSG</type>

...

<register>

<name>data_dll_status</name>

<size>8<\size>

...

<field>

<name>adc_done</name>

<ValRTLSignal>i_ddrdatadigtop.i_ddrdatacr</ValRTLSignal>

...

<field> <- Inconsistent structure

<name>adc_clock</name>

<ValRTLSignal>i_ddrdatadigtop.i_ddrdatacr_[1]</ValRTLSignal>

...

</field>

...

</field>

</register>

</registerFile>

Similarly, a ‘register’ appearing in the scope of another ‘register’ or a ‘registerFile’ appearing in the scope of another ‘registerFile’ are also treated as inconsistencies.

Example 3: Deviation from the expected order of hierarchies

<register> <- Inconsistent structure

<name>data_dll_status</name>

<size>8<\size>

...

<registerFile>  <- Inconsistent structure

<name>cnxsp_top/ddrphy_top[1]/ddrdata1_type</name>

<type>MSG</type>

...

<field>

<name>adc_done</name>

<ValRTLSignal>i_ddrdatadigtop.i_ddrdatacr</ValRTLSignal>

...

<field> <- Inconsistent structure

<name>adc_clock</name>

<ValRTLSignal>i_ddrdatadigtop.i_ddrdatacr_[1]</ValRTLSignal>

...

</field>

...

</field>

</registerFile>

</register>

#### Reserved CR Fabric Types Names

For some CR fabric types, special tool assumptions are made for the collateral information extraction. For example, special properties unique to a recognized CR fabric type are predefined by tool and their values are automatically extracted from CRIF files without user’s direction.

Specific names are reserved for the CR fabric types recognized by tool. It is required to use the reserved names in cr_spec section for the recognized CR fabric types. The following table shows the list of the recognized CR fabric types and their corresponding reserved names in cr_spec.

| CR fabric type | Reserved name in cr_spec |
| --- | --- |
| IOSF-SB | IOSF_SB |
| CRI | CRI |

#### Predefined Properties

For every register space in every CR fabric type, the following CR properties are predefined in tool and collected from CRIF by reading the corresponding attributes given in the table even without specification in ‘cr_spec’.

| Property | CR scope | Attribute name | Predefined property name | Value type | Required /Optional (default value) |
| --- | --- | --- | --- | --- | --- |
| Register file name | Register File | name | ‘REG_FILE_NAME | string | Required |
| Async reset signal |  | ResetSignal | ‘RF_ASYNC_RST_SIGNAL | string | Optional (“”) |
| Sync reset signal |  | SyncResetSignal | ‘RF_SYNC_RST_SIGNAL | string | Optional (“”) |
| Register name | Register | name | ‘REG_NAME | string | Required |
| Register size |  | size | ‘REG_SIZE | verilog_value | Required |
| Async reset signal |  | ResetSignal | ‘REG_ASYNC_RST_SIGNAL | string | Optional (“”) |
| Sync reset signal |  | SyncResetSignal | ‘REG_SYNC_RST_SIGNAL | string | Optional (“”) |
| Register field name | Field | name | ‘FIELD_NAME | string | Required |
| Field access type |  | access | ‘ACCESS | string | Required |
| Field description |  | description | ‘FIELD_DESC | string | Optional (“”) |
| Begin bit index for field |  | bitOffset | ‘FIELD_BEGIN_INDEX | verilog_value | Required |
| Field width |  | bitWidth | ‘FIELD_WIDTH | verilog_value | Required |
| RTL signal name for field |  | ValRTLSignal | ‘RTL_SIGNAL | string | Optional (“”) |
| Reset value |  | reset | ‘RESET_VALUE | verilog_value | Optional (’h0) |
| Async reset signal |  | ResetSignal | ‘FIELD_ASYNC_RST_SIGNAL | string | Optional (“”) |
| Sync reset signal |  | SyncResetSignal | ‘FIELD_SYNC_RST_SIGNAL | string | Optional (“”) |

A property name is assigned to each property with a reserved keyword so that the keyword can be used to reference the property in user interaction. Please note that all such reserved keywords begin with a tick (‘) in the beginning of the word. During CR sequence transformation, the value of the attribute corresponding to a property may be obtained using the property name keyword.

If a property defined in the above table is given with a dffierent attribute from the one listed here, the new attribute that maps to the given property should be specified in ‘cr_spec’ to override the attribute listed here. The syntax to redefine the mapping between a property and an attribute is described in ‘cr_spec’ section. Please note that the value type for a property should not be changed even if another attribute is mapped by user. If the new value type is not consistent with the one listed here, SPF will produce an error.

‘ResetSignal’ and ‘SyncResetSignal’ attributes may be given in register file, register or field level in a CRIF file. If they are specified in field level, the one given in field level will be assigned to the properties. If they are specified only in higher level than field, the one given in the lowest level will be assigned to the properties.

Additional set of properties are predefined for a specific fabric type such as IOSF-SB as described in the following subsection.

##### IOSF-SB

The following table shows the properties predefined for IOSF-SB fabric type.

| Property | CR scope | Attribute name | Predefined property name | Value type | Required /Optional (default value) |
| --- | --- | --- | --- | --- | --- |
| IOSF-SB Port ID | Register File | portid | ‘PORTID | verilog_value | Required |
| Register file type |  | type | ‘REG_FILE_TYPE | string | Required |
| Function ID |  | SB_Fid | ‘SB_FID | verilog_value | Optional (0) |
| Base Address |  | SB_Bar | ‘SB_BAR | verilog_value | Optional (0) |
| Opcode Mnemonic |  | opcode | ‘OPCODE | string | Required |
| Opcode value for read |  | N/A Tool populated. | ‘OPCODE_READ | verilog_value | Available for valid ‘OPCODE value |
| Opcode value for write |  | N/A Tool populated. | ‘OPCODE_WRITE | verilog_value | Available for valid ‘OPCODE value |
| Fabric type |  | Fabric | ‘FABRIC | string | Required |
| Address offset | Register | addressOffset | ‘ADDR_OFFSET | verilog_value | Required |

While ‘OPCODE populated from opcode attribute in register file scope in CRIF is defined as a string type property, it actually is an enum type property whose value is determined based on register access operation. User is expected to provide the mapping between the mnemonics of the property and the actual values per register access operation in ‘OPCODE_ENUM_MAP’ section in ‘cr_spec’ (please refer to “cr_spec” section). To support an access to the actual values of the string given in ‘OPCODE property, two additional properties are defined -  ‘OPCODE_READ for register read operation and ‘OPCODE_WRITE for register write operation.

##### CRI

The following table shows the predefined properties for CRI fabric type.

| Property | CR scope | Attribute name | Predefined property name | Value type | Required /Optional (default value) |
| --- | --- | --- | --- | --- | --- |
| Address offset | Register | addressOffset | ‘ADDR_OFFSET | verilog_value | Required |

#### cr_spec

For each register space of interest, if additional CR properties which are not part of the predefined ones need to be defined and their values need to be extracted from specific attributes in CRIF files, they should be defined in ‘cr_spec’ section. The mapping between a predefined property and an attribute may be redefined here as well.

The format of ‘cr_spec’ is described as follows. For each CR fabric type, the properties common to all the register spaces under the given CR fabric type are defined in CR fabric type section. The common properties are defined in REQUIRED and OPTIONAL subsections. The properties whose corresponding attributes should always exist in CRIF file are defined in REQUIRED subsection. The other properties are defined in OPTIONAL subsection. Register space specific information is defined per each register space in ‘REGISTER_SPACES’ subsection under the given CR fabric type section. In addition to REQUIRED, OPTIONAL and REGISTER_SPACES subsections, fabric type specific information may be defined in a CR fabric type section.

In REGISTER_SPACES section, a register space subsection is defined per each register space. In a register space subsection, the properties unique to the given register space and the register files selection condition for the register space are defined. The register files selection condition is defined in QUALIFY subsection which is mandatory for every register space. Similar to a fabric type section, the properties unique to the given register space are defined in the subsections, REQUIRED and OPTIONAL, which are optional. REQUIRED and OPTIONAL subsections within a register space section extend or override the common properties definitions given in the CR fabric type level.

project_spec {

cr_spec {

<CR fabric type> {

REQUIRED {

<property>{

spec = REG_FILE::<attr>|REG::<attr>|FIELD::<attr>;

[type = string|verilog_value|enum;]

[mnemonic {<mnemonic name> = <value>, ...}]

}

...

}

OPTIONAL {

<property> {

spec = REG_FILE::<attr>|REG::<attr>|FIELD::<attr>;

[type = string|verilog_value|enum;]

[default = <default value>;]

[mnemonic {<mnemonic name> = <value>, ...}]

}

...

}

//	CR fabric specific definition

REGISTER_SPACES {

<register space> {

QUALIFY {

A: <property> = <value>;

[B: <property> = <value>;]

...

CONDITION: <logical expression (with !,&&,||) of A, B, ...>;

}

REQUIRED {

<property> {

spec = REG_FILE::<attr> | REG::<attr> |

FIELD::<attr>;

[type = string|verilog_value|enum;]

[mnemonic {<mnemonic name> = <value>,

...}]

}

...

}

OPTIONAL {

<property> {

spec = REG_FILE::<attr> | REG::<attr> |

FIELD::<attr>;

[type = string|Verilog_value|enum;]

[default = <default value>;]

[mnemonic {<mnemonic name> = <value>,

...}]

}

...

}

}

...

}#end of REGISTER_SPACES

}

}#end cr_spec

}#end project_spec

##### REQUIRED

In REQUIRED subsection under a CR fabric type section, user may define new properties common to all the register spaces under the given CR fabric type, which are not part of the SPF predefined ones, if the extractions of their values are necessary and the attributes corresponding to them are required to be available in CRIF file. Please note that the properties defined in this section are ‘required’ ones in a sense that the corresponding attributes should always exist in CRIF file and valid values should be obtained from them.

Also, predefined properties may be redefined here by assigning new ‘required’ attributes different from the ones assumed by tool especially when CRIF file format changes. Please note that a predefined property name should have a leading tick symbol while a user defined property name should not start with a special character. The predefined properties which are redefined here become ‘required’ properties even though they are not considered as ‘required’ in the default tool assumption.

If the attributes mapped to the properties defined here do not exist in the given CRIF file, or the values obtained from them are not valid, tool will report an error. The scope of a given attribute can be specified using the keywords ‘REG_FILE’, ‘REG’ or ‘FIELD’. For example, REG_FILE::type refers to ‘type’ attribute in the register file hierarchy and FIELD::reset refers to ‘reset’ attribute in the field hierarchy.

If the value of a given attribute can be represented in Verilog value format (integer or <# bits>’b|d|h<number> or 0X<number>), the type of the value is not required to be specified. Otherwise, a proper value type should be provided. The supported value types are Verilog value, string and enum type. If the type of a value is enum, the mapping of each enumerated name to a constant value should be provided in ‘mnemonic’ field. While overriding a predefined property definition with a different attribute it is prohibited to change the value type from the default tool assumption.

##### OPTIONAL

In OPTIONAL subsection under a CR fabric type section, user may define new properties common to all the register spaces under the given CR fabric type, which are not part of the SPF predefined ones, if the extractions of their values are necessary and the attributes corresponding to them may or may not be given in CRIF.

In addition to an attribute and a value type specification, a default value may be provided for a property defined here. If the attribute mapped to the property defined here does not exist in the given CRIF file, the default value will be assumed for the given property. If no default value is provided in spec, then the tool will assign a default value based on the type of the property. For the properties of type ‘verilog_type’ the default value ‘b0 will be assigned, and for the properties of type ‘string’ the default value  “” (null string) will be assigned. For the properties of type ‘enum’, it is mandatory that user specifies the mnemonic field. Otherwise an error will be thrown. A default value must be specified for the optional properties of type ‘enum’.

Also, predefined properties may be redefined here in order to assign a new attribute or default value different from the one assumed by tool. This allows users to reconfigure the predefined properties as CRIF specifications change. Such predefined properties become ‘optional’ even though they were originally defined as ‘required’ in the default tool assumption. When redefining a predefined property, user is not allowed to change the type of the property. The type field is optional but, if specified, it should be consistent with the default tool assumption.

##### Fabric Type Specific Information

For the CR fabric types recognized by tool, additional user inputs are defined for interpreting and processing the CR collateral information specific to the given fabric types.

###### IOSF-SB

The following information is required to be provided in ‘IOSF_SB’ fabric type section.

Opcode Enumeration

For IOSF_SB fabric type, even though ‘OPCODE property is predefined as string type in tool, the string given to the property should be treated as a mnemonic and the actual value corresponding to the mnemonic can be identified per register access operation type. The mapping between a mnemonic of the property and its corresponding value is not defined in tool and may change from product to product. So it is required to provide the mapping information in OPCODE_ENUM_MAP section. The syntax of the section is given below.

cr_spec {

IOSF_SB {

REQUIRED { ... }

OPTIONAL { ... }

OPCODE_ENUM_MAP {

<mnemonic> = <value> (read) : <value> (write);

...

}

REGISTER_SPACES { ... }

}

...

}

Please note that two different value mappings are required, one for register read operation and the other for register write operation. The following shows an example of OPCODE_ENUM_MAP for IOSF_SB fabric type.

cr_spec {

IOSF_SB {

REQUIRED { ... }

OPTIONAL { ... }

OPCODE_ENUM_MAP {

MEM-SB = ‘h00 (read) : ‘h01 (write);

IO-SB  = ‘h02 (read) : ‘h03 (write);

CFG-SB = ‘h04 (read) : ‘h05 (write);

CR-SB  = ‘h06 (read) : ‘h07 (write);

}

REGISTER_SPACES { ... }

}

...

}

##### REGISTER_SPACES

In REGISTER_SPACES subsection, register space specific information is provided per each register space. Each register space name should be unique across all the fabric types defined in ‘cr_spec’. Each register space section should include QUALIFY subsection, and it may include REQUIRED and OPTIONAL subsections optionally.

###### QUALIFY

In QUALIFY subsection, which must be defined for each register space section, the condition to determine whether a register file belongs to the corresponding register space is specified.

To formulate the condition, user may list register file properties along with their expected values. Users can use regular expressions to specify the expected values. The regular expression needs to be enclosed in slashes ‘/’.  The properties used in the condition may be those predefined in tool, or explicitly defined in the given CR fabric type section or in the given register space section. Regardless of the source, all those properties should be ‘required’ ones in the given register space perspective. The attribute corresponding to each property must exist in CRIF and a valid value should be obtainable for each property. Otherwise, tool will produce an error message. The actual qualification condition is specified in CONDITION field as a logical expression of the properties listed in this section. The logical operators allowed in CONDITION field are !(not), ||(or) and &&(and).

An example is given below.

...

QUALIFY {

A: ‘REG_FILE_TYPE = “MSG”;

B: ‘FABRIC = “GPSB”;

CONDITION: A && B;

}

...

Example using regular expressions:

...

QUALIFY {

A : 'REG_FILE_NAME = /gnrio_System/fusetop/fusectl_reg/.*/ ;

CONDITION : A;

}

...

In this example, all register files in the crif xml starting with “gnrio_System/fusetop/fusectl_reg/” qualify and are stored under the register space. gnrio_System/fusetop/fusectl_reg/M_BAR4, gnrio_System/fusetop/fusectl_reg/M_BAR3 are list of register files selected using the regular expression

For a register file to be qualified as part of the given register space whose qualification condition is given above, the value of the attribute associated with ‘REG_FILE_TYPE must be equal to “MSG” and the value of the attribute associated with ‘FABRIC must be equal to “GPSB”.

Note: If a register file qualifies for multiple registers spaces, then the tool will maintain the reference to the same register file instance under all of the register spaces. In other words, if the register file is targeted for register transaction under a specific register space and the register values are updated, then this change will be visible in all of the other register spaces to which the register file qualified.

###### REQUIRED in Register Space

REQUIRED subsection may be defined under each register space section if ‘required’ properties specific to the given register space need to be defined. The properties defined under the CR fabric type section may be redefined here. If this happens, the properties defined under the given register space section will override the definition given at the CR fabric type section. The properties defined in OPTIONAL subsection under the CR fabric type section may be redefined here as ‘required’ properties. The final set of ‘required’ properties of a register space will be the union of the properties defined in REQUIRED subsection under the given register space and the ones defined in REQUIRED subsection under the CR fabric type section which are not redefined under the given register space.

Predefined properties may be redefined here. The predefined properties which are redefined here become ‘required’ properties even though they are not considered as ‘required’ in the default tool assumption. However, the value type should be consistent with the default tool assumption.

###### OPTIONAL in Register Space

OPTIONAL subsection may be defined under each register space section if ‘optional’ properties specific to the given register space need to be defined. The properties defined under the CR fabric type section may be redefined here. If this happens, the properties defined under the given register space section will override the definition given at the CR fabric type section. The properties defined in REQUIRED subsection under the CR fabric type section may be redefined here as ‘optional’ properties. The final set of ‘optional’ properties of a register space will be the union of the properties defined in OPTIONAL subsection under the given register space and the ones defined in OPTIONAL subsection under the CR fabric type section which are not redefined under the given register space.

Predefined properties may be redefined here. Such predefined properties become ‘optional’ even though they were originally defined as ‘required’ in the default tool assumption. When redefining a predefined property, the value type should be consistent with the default tool assumption.

##### Example

An example of ‘cr_spec’ is given below.

      - QUALIFY subsection within the ‘GPSB’ register space lists predefined properties ‘REG_FILE_TYPE and ‘FABRIC with their expected values as “MSG” and “GPSB”, respectively. This indicates that any register file, which has attribute ‘type’ with value “MSG” and attribute ‘Fabric’ with value “GPSB”, will be classified as a register file belonging to GPSB register space.

      - In the REQUIRED subsection under IOSF_SB fabric type, PREFIX, MEMADDRLEN, OPMODE and DESC properties are defined. In the REQUIRED subsection under ‘GPSB’ register space, a register file level property TRUST is defined and a predefined property ‘RTL_SIGNAL is redefined. The final set of required properties for ‘GPSB’ register space are PREFIX, MEMADDRLEN, OPMODE, TRUST and ‘RTL_SIGNAL. Note that DESC becomes an optional property in ‘GPSB’ register space since it is redefined in the OPTIONAL subsection under ‘GPSB’ register space.

      - In the OPTIONAL subsection in the IOSF_SB fabric type, predefined properties ‘SB_BAR and ‘SB_FID are redefined. For ‘SB_BAR and ‘SB_FID, the corresponding attributes (given in “Predefined Properties” section) are replaced with “SB_bar” and “SB_fid” and their default values are redefined as 4 and 3, respectively.

      - Notice that ‘PMSB’ register space section does not list an OPTIONAL subsection so the optional properties for ‘PMSB’ register space are inherited from the IOSF_SB fabric type section.

project_spec {

cr_spec {

IOSF_SB {

OPCODE_ENUM_MAP {

MEM-SB = ‘h00 (read) : ‘h01 (write);

IO-SB  = ‘h02 (read) : ‘h03 (write);

CFG-SB = ‘h04 (read) : ‘h05 (write);

CR-SB  = ‘h06 (read) : ‘h07 (write);

}

REQUIRED {

PREFIX {

spec = REG_FILE::prefix;

type = string;

}

MEMADDRLEN {

spec = REG_FILE::MEMAddrLen;

type = verilog_value;

}

OPMODE {

spec = REG_FILE::OperationMode;

type = verilog_value;

}

DESC {

spec = REG::description;

type = string;

}

}

OPTIONAL {

‘SB_BAR { spec = REG_FILE::SB_bar; default = 4; }

‘SB_FID { spec = REG_FILE::SB_fid; default = 3; }

}

REGISTER_SPACES {

GPSB {

QUALIFY {

A: ‘REG_FILE_TYPE = “MSG”;

B: ‘FABRIC = “GPSB”;

CONDITION: A && B;

}

REQUIRED{										  			TRUST {

spec = REG_FILE::RequiresTrust;

type = verilog_value;

}

‘RTL_SIGNAL {

spec = FIELD::ValRTLSignal;

}

}

OPTIONAL{

DESC {

spec = REG::description;

type = string;

}

}

}

PMSB {

QUALIFY {

A: ‘REG_FILE_TYPE = “MSG”;

B: ‘FABRIC = “PMSB”;

CONDITION: A && B;

}

REQUIRED{										  			‘FABRIC {

spec = REG_FILE::Fabric;

type = string;

}

}

}#end of PMSB spec

}#end of register space spec definitions

}

}

}

### STF2TAP Specification

STF2TAP or STF2IJTAG bridge mapping information should be provided in STF2TAP_mapping section of project_spec. In STF2TAP_mapping, each bridge is defined in STF2TAP_bridge sub-section with a unique bridge name. No duplicate bridge definitions are allowed in STF2TAP_mapping. In STF2TAP_bridge section, the placement of the bridge in TAP network is provided under TAP field. The TAP field can consist of TAP wrap, TAP or register.

The STF endpoint where the bridge is connected is provided under STF field. The mapping information of necessary bridge fields to actual STF register fields is provided under STF subsection. Multiple bridges can exist under the same STF endpoint. In such case, care should be taken that the control registers defined under each bridge is unique.  All the necessary bridge fields for STF2TAP bridges are defined in “STF2TAP Mapping” section. All the necessary bridge fields for STF2IJTAG bridges are defined in “STF2IJTAG Mapping” section.

The syntax of STF2TAP_mapping is given below.

project_spec {

STF2TAP_mapping {

STF2TAP_bridge <bridge_name1> {

TAP : [<TAP_NETWORK>::]<Tap_Wrap_name> | <TAP_name> | <TAP_name/reg_file_name => TAP_reg_name>;

STF : <endpoint_name> {

<STF2TAP bridge field> = <STF reg field>; | <STF2IJTAG bridge field> = <STF reg field>;

...

}

STF2TAP_bridge <bridge_name2> {

TAP : [<TAP_NETWORK>::] Tap_Wrap_name> | <TAP_name> | <TAP_name/reg_file_name => TAP_reg_name>;

STF : <endpoint_name> {

<STF2TAP bridge field> = <STF reg field>; | <STF2IJTAG bridge field> = <STF reg field>;

...

}

...

}

}

<STF2TAP_bridge_field>: shown in the table in “STF2TAP Mapping” section

<STF2IJTAG_bridge_field>: shown in the table in “STF2IJTAG Mapping” section

#### STF2TAP Mapping

The description of each STF2TAP bridge field is given as follows.

| STF2IJTAG Bridge Field | Description |
| --- | --- |
| MODE | bridge enable and mode configuration. |
| CYCLE_PERIOD | In direct mode, it will be the width of a TCK clock phase in terms of the number of STF clock cycles. In “clocked” and “shift” modes, clock period is clock ratio between STF and TCK clock. |
| CLOCK_CONTROL | TCK clock phase width in terms of the number of STF clock cycles |
| CLOCK_50P_DUTY | A TCK clock period is made up of 4 phases if this mode is on. Otherwise, it is made up of 3 phases. |
| TRST | STF field mapped to TAP “trstb” port. |
| TCK | STF field mapped to TAP “tck” port. |
| TMS | STF field mapped to TAP “tms” port. |
| TDI_TDO | STF field mapped to TAP “tdi” port as well as “tdo” port. |
| SHIFT_COUNT | Only used in shift mode. Set the number of additional tdi bits to drive after the very first tdi data given in TDI_TDO field. |
| SHIFT_DATA | Only used in shift mode. Set the additional tdi and tms bits to drive after the very first tdi/tms data given in TDI_TDO/TMS field. |
| EXPECT | Optional STF field for EXPECT data, which is required when stf2Tap_input_expect_mode is set |
| MASK | Optional STF field for MASK data, which is required when stf2Tap_input_expect_mode is set |

Please note that TMS, TDI, and TDO are unqiue to STF2TAP. They are only allowed in STF2TAP bridges. Tool will throw an error if they appear in STF2IJTAG bridges.

An example of STF2TAP bridge mapping definition is given as follows.

project_spec {

STF2TAP_mapping {

STF2TAP_bridge bridge0 {

TAP: TAP_WRAP1;

STF: STF_EP1 {

MODE : STF2TAP_CONTROL->mode;

CYCLE_PERIOD : STF2TAP_CONTROL->cycle_period;

CLOCK_CONTROL : STF2TAP_CONTROL->clock_control;

CLOCK_50P_DUTY: STF2TAP_CONTROL->clock_50p;

TRST: STF2TAP_BRIDGE->trst_b;

TCK: STF2TAP_BRIDGE->tck;

TMS: STF2TAP_BRIDGE->tms;

TDI_TDO: STF2TAP_BRIDGE->tdi_tdo;

SHIFT_COUNT: STF2TAP_BRIDGE->shift_count;

SHIFT_DATA: STF2TAP_BRIDGE->shift_data;

EXPECT: EXPECT;

MASK: MASK;

}

}

STF2TAP_bridge bridge1 {

TAP : cpk_debug_tap_mng_cpk_nac_c0_r3;

STF : clst_scan_punit_p {

MODE : STF2TAP_CONTROL->mode;

CYCLE_PERIOD : STF2TAP_CONTROL->cycle_period;

CLOCK_CONTROL : STF2TAP_CONTROL->clock_control;

CLOCK_50P_DUTY: STF2TAP_CONTROL->clock_50p;

TRST: STF2TAP_BRIDGE->trst_b;

TCK: STF2TAP_BRIDGE->tck;

TMS: STF2TAP_BRIDGE->tms;

TDI_TDO: STF2TAP_BRIDGE->tdi_tdo;

SHIFT_COUNT: STF2TAP_BRIDGE->shift_count;

SHIFT_DATA: STF2TAP_BRIDGE->shift_data;

}

}

STF2TAP_bridge bridge2 {

TAP : mbist_tap_cif_cpk_nac_c0_r3;

STF : clst_scan_iop {

TCK: STF2TAP_BRIDGE->tck;

TMS: STF2TAP_BRIDGE->tms;

TDI_TDO: STF2TAP_BRIDGE->tdi_tdo;

SHIFT_COUNT: STF2TAP_BRIDGE->shift_count;

SHIFT_DATA: STF2TAP_BRIDGE->shift_data;

MODE : STF2TAP_CONTROL->mode;

CYCLE_PERIOD : STF2TAP_CONTROL->cycle_period;

CLOCK_CONTROL : STF2TAP_CONTROL->clock_control;

CLOCK_50P_DUTY: STF2TAP_CONTROL->clock_50p;

TRST: STF2TAP_BRIDGE->trst_b;

}

}

}

}

#### STF2IJTAG Mapping

Will be added in future.

## HVM Rule Specification

All specifications for HVM purpose are contained in hvm_rule_spec which is defined per project. Today, the Patmod Support specification can be specified in hvm rule Spec

### Patmod Specification

Pattern modification definitions are given in Patmod Spec. The various patmod variable definitions to be applied to a project can be specified in Patmod Spec section.

#### Patmod variable definition

There can be multiple Patmod variable definitions in one Patmod section. Each definition can include the optional description for the variable, optional variable usage restrictions and the mapping of symbolic names to the values which is mandatory. The syntax for a variable definition in Patmod spec is as shown:

hvm_rule_spec {

patmod_spec {

<var_name> {

[desc: “<description>”;]

[usage {

[<tap register field spec>];

...

}]

values {

<patmod name> = <Verilog value>;

...

}

} //var-name spec

} //patmod spec

}  //hvm rule spec

<tap register field spec>:= <tap name> | <tap name>=><register name>>[<bit range>,....,<bit range>]| <tap name>=><register name>-><alias name>[<bit range>,....,<bit range>]

<Verilog value> = [<size>]<’b[<1|0|X>]+|’h[<0-9|A-F>]+|’d[<0-9>]+|[0-9]+>

- Desc: Optional. The description for the variable can be specified here.

- Usage: Optional. The restrictions for the variable usage can be specified in the usage section. User may set the list of register fields in which the variable usage is restricted. The list can contain an exhaustive list of all register fields where the Patmod variable can be applied. For non contiguous bits specification, bit ranges separated by comma can be provided.  The field range specified should be MSB:LSB where MSB > LSB. Also the field ranges should be specified in decreasing order .i.e., MSB of second non-contiguous bit range should be less than LSB of first bit range.The usage of the variable in the patmod command can be a subset of the field specified here in the usage section

- Values: Mandatory. The mapping from the symbolic names to the values should be specified in this section. User may restrict the patmod name and value usage to input only or output only. By default, the value can be used for both input and output.

## Tool Option Setting

Tool options defining the overall tool behavior should be given in template file.

Note: Tool options are different from on-the-fly directives. A tool option can not be specified in input test sequence because it is not supposed to be changed in the middle of the input test sequence. On the other hand, on-the-fly directives are defined only for the options or modes which can be changed at any point of the input test sequence.

Default value for an on-the-fly directive may be set in the template file as well. The effect of setting a default value of a directive in the template file is the same as setting the directive with the given option in the beginning of input test sequence.

### Tool Option

Tool options can be given in the section defined below.

tool_setup_options {

tool_set <option name> <option>;

…

}

The options are tabulated for quick reference and and they are described below.

| Tool option | Impact |
| --- | --- |
| print_itpp_header | Turns on/off ITPP header information. By default, the option is on |
| top_tap | Specifies root tap for content generation at intermediate hierarchy |
| top_stf | Specifies target STF hierarchy level for generating content at intermediate hierarchy level of STF. |
| tap_2_stf | Specifies the entry and exit components to a partial STF network in TAP2STF mode. |
| handle_duplicate_label | Defines behavior of tool on how to handle duplicate labels |
| convert_internal_remark_to_comment | Turns on/off to convert spf generated remark to comment. By default is off. |
| allow_multiple_register_per_opcode | Multiple registers can access same op-code. By default if off. |
| stf_add_underscore_to_vector | Add understores at STF vector field boundary. |
| assert_stf_fields_in_valid_out_packets | Assert additional fields in STF OUT packets with valid data. |
| tap_prefix | Specifies JTAG port keyword and its prefix to indicate TAP transactions on the given JTAG port. The prefix specified will be prepended to scani, scand, pscand and to_state commands in itpp. |
| record_expected_data_in_outputValidPacketFile | During scan pattern transformation, records expected scan out data in output valid packet file |
| stf_skip_bypass_hub_error_check | Skip error check for an unreachable endpoint access. |
| tap_ignore_name_case_sensitivity | Case sensitivity of TAP related names among the test sequence file, Map file, checkpoint file, collateral file will be ignored. The option is only valid for TAP commands. |
| cr_error_on_inconsistent _crif | Inconsistencies in CRIF format will be reported as errors instead of warnings |
| vector_compression | Compress repeated STF ITPP vectors into a single vector with repeat count |
| assert_stf_fields_only_for_user_compare_pkt | When turned on, the tool will not assert STF output packet fields(GID, OV or OPCODE) if the packet data is filled with only don’t cares (X). This option can only be used in conjunction with the tool option ‘assert_stf_fields_in_valid_out_packets’. The default value for this option is ‘off’. |
| flush_end_of_seq_non_scan_output_packets | This option is effective during the end of sequence flush operation when input valid packet is given. When turned on, the tool will flush all output packets irrespective of whether the packet contains scan data. In default behavior (off), the tool will only flush output packets only until the last packet with valid scan data and the remaining packets will be discarded. The default value for this option is ‘off’. |

#### print_itpp_header

Printing ITPP header information may be enabled or disabled using this option. By default, the option is on.

tool_set print_itpp_header <on|off>;

#### top_tap

This option sets the top level tap for the tap network in case generating content at intermediate hierarchy level is desired. This option overrides the top-level tap defined in the TOPOLOGY section in the tap collateral file and the content will be generated at the top-level tap defined by this option. This option is mandatory if incomplete TAP design collateral spec is given. Content generation at intermediate hierarchy for IJTAG network is not supporte in SPF.

tool_set top_tap <tap name>;

#### top_stf

This option sets the target STF hierarchy at which final content is desired to be generated by SPF after transformation. This option will be used for generating content at intermediate hierarchy level of STF network.

This option is mandatory if incomplete STF design collateral spec is given. Please refer to “Incomplete Collateral Spec” section of “Pattern Framework Spec – Design Collateral Spec” document.

tool_set top_stf <inout component> | <input component>, <output component>;

If input entry to STF network is different from output component of the network, both input STF component and output STF component should be specified. Both input and output components should belong to the same ring and should be at same level. Output should apprear after Input in hierarchy order specified in collateral file. If the same STF component serves as both input entry and output exit point from STF, a single component will be given.

#### tap_2_stf

This option sets the target STF components as the entry and exit points into the STF network. It enables content generation at the intermediate hierarchy level of STF network similar to top_stf option. However, the generated content is delivered through the entry TAP and read at exit TAP. Entry/exit TAP information is specified in an STF component to TAP map file. The final content generated by SPF is TAP test content. This option is mandatory to recognize TAP2STF mode, when there is a need to deliver test content to a partial STF network through a TAP network. Even though this option is optional for reverse mapping, it is recommended to give this option regardless of forward transformation or reverse mapping mode. In reverse mapping for TAP2STF mode, a commandline option for specifying TCK cycles in TAP FSM transitions must be given. Please refer to “TCK Cycles for TAP FSM Transition” section of “PatternTransformationAndReverseMapping” document.

The syntax for the tap_2_stf option is similar to the top_stf option, where either a single STF component or two STF components can be specified to serve as the input and output points to the STF network.

tool_set tap_2_stf [inout_component  |  in_component, out_component>];

#### handle_duplicate_label

By default, SPF allows only unique labels and errors out in case duplicate labels are given in the test sequence or there are duplicate labels produced as a result of internal label generation. However, in some cases it may be required to allow duplicate labels in the test sequence and the syntax of the tool set up option in the template file is given below –

tool_set handle_duplicate_label  <DEFAULT|ALLOW|UNIQUIFY>;

The three modes are described below -

- DEFAULT: This is the default mode where SPF produces an error if there are any duplicate labels in the itpp which may be as a result of duplicate labels in test sequence or due to duplication as a result of internal label generation that matches with a test sequence label.

- ALLOW: In this mode, if there are duplicate labels in the itpp, a warning is produced and there is no change in the label, so there will be duplicate labels printed to the itpp. Note that this option cannot be set when either valid packet creation is required or during reverse mapping.

- UNIQUIFY: In this mode, in case there are duplicate labels produced in itpp, they are uniquified by appending a label id and the uniquified label is reported to itpp.

The label uniquification is described in “Pattern Framework Spec – Test” document.

#### convert _internal_remark_to_comment

By default, SPF auto generate remark in SPF itpp. Convert_remark_to_comment option allows user to convert all auto-generated remark (rem:) to comment (# ). The option however doesn’t impact the remark feature in comment command.

tool_set convert_internal_remark_to_comment <on | off>;

For instance:

In Template file:

tool_setup_options {

tool_set convert_internal_remark_to_comment <on | off>;

}

In test sequence file:

This test sequence shows remark in comment command doesn’t impact by this tool option. Moreover, all auto-generated remarks in stf and tap will be converted to comment statement.

comment rem "Starting with EP5 endpoint";

comment "selecting EP5 and set to gId 1";

set_stf_packet input SELECT 1 'h8 0;

comment rem "turn on EP5 output enable";

focus_stf EP5;

set MODE->gid0_output_enable = 'b1;

flush;

comment rem "context switching from stf to tap";

focus_tap sTAPa;

set sTAPa=>reg_b = 'b10111011;

flush;

Output itpp file:

rem: Starting with EP5 endpoint

# selecting EP5 and set to gId 1

# in: stf_in : GID = 1 OPCODE = SELECT PID = 1000 GID Bank = 0

vector: stf_in(00000000000000000010000001010001) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: turn on EP5 output enable

# in: REG = MODE STOPNAME = EP5 stf_in : GID = 1 OPCODE = CONTROL Payload = 00100

vector: stf_in(00000000000000000001000001101001) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

# in: REG = MODE STOPNAME = EP5 stf_in : GID = 1 OPCODE = DATA Payload = 00000001

vector: stf_in(00000000000000000000010001101101) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: context switching from stf to tap

# IR_REGISTERS: TAP_CL=>SEL_SELECT_REG[8] = 8 bits ;

scani: 10000111 ;

to_state: Run-Test/Idle ;

# DR_SHIFT: TAP_CL=>select_reg[6] = 6 bits ;

scand: 000001, XXXXXX ;

to_state: Run-Test/Idle ;

# IR_REGISTERS: TAP_CL=>bypass[8] + sTAPa=>SEL_COMPLEX1[8] = 16 bits ;

scani: 11111111_01000100 ;

to_state: Run-Test/Idle ;

# DR_SHIFT: TAP_CL=>bypass[1] + sTAPa=>reg_c[5] + sTAPa=>reg_b[8] = 14 bits ;

scand: 0_10000_10111011, X_XXXXX_XXXXXXXX ;

to_state: Run-Test/Idle ;

# IR_REGISTERS: TAP_CL=>SEL_SELECT_REG[8] + sTAPa=>bypass[8] = 16 bits ;

scani: 10000111_11111111 ;

to_state: Run-Test/Idle ;

# DR_SHIFT: TAP_CL=>select_reg[6] + sTAPa=>bypass[1] = 7 bits ;

scand: 000000_0, XXXXXX_X ;

to_state: Run-Test/Idle ;

#### stf_add_underscore_to_vector

By default, no field separator is added in the middle of STF input/output vectors in ITPP. With this option, ‘_’ is added at field boundaries of the final packet used in STF vector in ITPP. Note that no underscore will be added to dummy output packets.

Syntax:

tool_set stf_add_underscore_to_vector <on|off>;

The following example shows a case where an STF packet is used for an ITPP vector.

In Test Sequence File

set_stf_packet input 'd4 1 'b0100 1;

focus_stf HUB1;

set ChildGroupMemberships->group_2 = 'b1;

compare ChildGroupMemberships->group_2 = 'b1;

flush;

ITPP File:

rem: in: stf_in : GID = 1 OPCODE = SELECT PID = 0100 GID Bank = 1

vector: stf_in(00000000000000000_1_0100_0001_0100_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: REG = ChildGroupMemberships STOPNAME = HUB1 stf_in : GID = 1 OPCODE = CONTROL Payload = 00011

vector: stf_in(0000000000000000000011_0001_1010_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: REG = ChildGroupMemberships STOPNAME = HUB1 stf_in : GID = 1 OPCODE = DATA Payload = 111

vector: stf_in(0000000000000000000111_0001_1011_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: REG = ChildGroupMemberships STOPNAME = HUB1 stf_in : GID = 1 OPCODE = DATA Payload = 111

vector: stf_in(0000000000000000000111_0001_1011_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: REG = ChildGroupMemberships STOPNAME = HUB1 stf_in : GID = 1 OPCODE = DATA Payload = 111

vector: stf_in(0000000000000000000111_0001_1011_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: stf_in : GID = 1 OPCODE = NOP Payload =

vector: stf_in(0000000000000000000000_0001_0000_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: stf_in : GID = 1 OPCODE = NOP Payload =

vector: stf_in(0000000000000000000000_0001_0000_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: stf_in : GID = 1 OPCODE = NOP Payload =

vector: stf_in(0000000000000000000000_0001_0000_0_1) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: stf_in : GID = 1 OPCODE = NOP Payload =

rem: out: REG = ChildGroupMemberships STOPNAME = HUB1 stf_out : GID = 1 OPCODE = DATA Payload = 1XX

vector: stf_in(0000000000000000000000_0001_0000_0_1) stf_out(XXXXXXXXXXXXXXXXXXXHXX_XXXX_XXXX_X_X);

rem: in: stf_in : GID = 1 OPCODE = NOP Payload =

rem: out: REG = ChildGroupMemberships STOPNAME = HUB1 stf_out : GID = 1 OPCODE = DATA Payload = 1XX

vector: stf_in(0000000000000000000000_0001_0000_0_1) stf_out(XXXXXXXXXXXXXXXXXXXHXX_XXXX_XXXX_X_X);

rem: in: stf_in : GID = 1 OPCODE = NOP Payload =

rem: out: REG = ChildGroupMemberships STOPNAME = HUB1 stf_out : GID = 1 OPCODE = DATA Payload = 1XX

vector: stf_in(0000000000000000000000_0001_0000_0_1) stf_out(XXXXXXXXXXXXXXXXXXXHXX_XXXX_XXXX_X_X);

The following example shows a case where STF test port packets are used for ITPP vectors.

set_stf_packet input 'd4 1 'b0100 1;

focus_stf STF_Test_Port_Name;

set RegisterTpA[4:5] = 'b11;

compare RegisterTpA[4:5] = 'b11;

flush;

ITPP File:

rem: in: tp_in : STF_CODE = 100 GROUP = 001 STF_DATA = 00000000000000010100 CONTROL_BITS = 11

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 01

vector: tp_in(11_00_00000000000000010100_001_0_100_00) tp_out(LH_XXX_XXXXXXXXXXXXXXXXXXX_XXX_XXX_XXX);

rem: in: tp_in : REG_INDEX = 00001 REG_DATA = 101110000 CONTROL_BITS = 10 STF_CODE = 10 REG = RegisterTpA TESTPORT = STF_Test_Port_Name

vector: tp_in(01_0000000_0000000101110000_0_10_10000) tp_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 100 GROUP = 000 STF_DATA = 00000000000000001011 CONTROL_BITS = 11 TESTPORT = STF_Test_Port_

Name

vector: tp_in(11_00_00000000000000001011_000_0_100_00) tp_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 100 GROUP = 000 STF_DATA = 00000000000000001011 CONTROL_BITS = 11 TESTPORT = STF_Test_Port_Name

vector: tp_in(11_00_00000000000000001011_000_0_100_00) tp_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: tp_in : STF_CODE = 100 GROUP = 000 STF_DATA = 00000000000000001011 CONTROL_BITS = 11 TESTPORT = STF_Test_Port_Name

rem: out: tp_out : REG_INDEX = 00001 REG_DATA = XXX11XXXX CONTROL_BITS = 10 REG = RegisterTpA TESTPORT = STF_Test_Port_Name

vector: tp_in(11_00_00000000000000001011_000_0_100_00) tp_out(HL_XXXXXXX_HHXXXX_XXXXXXXXXXX_LLLLH_XX);

#### allow_multiple_register_per_opcode

By default, SPF does not allow multiple registers accessing the same op-code. However, in some situations this may be unavoidable and this option can be used to get around the tool assumption.

This option is not allowed in SoC TAP network.

Syntax:

tool_set allow_multiple_register_per_opcode <on |off>;

The following are the tool assumptions and expectations:

- Concatenation of registers for an op-code is not allowed in this mode

Ex: OPCODE_REGISTERS : ‘b10110111 : reg_a + reg_b; is not allowed

- In a given flush block, registers with the same op-code cannot be accessed

- Registers having the same op-code are treated independently in this mode and so in the incremental  mode, the register values will be independently updated.

- The user is expected to access the right register, because in some cases the register defined in the collateral may not be physically present in the tap.

- When multiple TAP networks are handled in concurrent testing and both TAP link network and SoC TAP network coexist, this option applies only to TAP link network. It does not affect SoC TAP network. In other words, while an SoC TAP network is targeted, tool will behave as if the option were not given or turned off.

#### assert_stf_fields_in_valid_out_packets

This option enables assertion of fields, other than payload, in an STF output packet. It affects only valid output packets - output packets from an output enabled STF STOP in response to incoming packets. Today, only GID, OPCODE, and OUTPUT_VALID fields are suppoted. Syntax is given below.

tool_set assert_stf_fields_in_valid_out_packets <space separated fields>;

Example:

tool_set assert_stf_fields_in_valid_out_packets GID OPCODE OUTPUT_VALID;

Please refer to “Output Packet Field Value Control” section of “Pattern Framework Spec - Test” document for usage examples.

#### Tap_prefix

This option is used to set JTAG port keyword and the corresponding prefix for scani, scand, pscand and to_state commands in itpp. Syntax is shown below.

tool_set tap_prefix <JTAG port keyword>=”<prefix>”;

Example:

tool_set tap_prefix sec = “sec_” ;

Please refer to “Concurrent Testing with Multiple JTAG Ports” section of “Pattern Framework Spec - Test” document for usage examples.

#### record_expected_data_in_outputValidPacketFile

During scan content transformation, it is expected to produce output valid packet info file (with commandline option “--outputValidPacketFile”) so that later tester pattern failure on silicon can be easily mapped to the original scan cell location during reverse mapping. With this option, scan out data (expected data) will be captured in the output valid packet info file so that it can be compared with actual failing value from silicon during scan pattern reverse mapping as a basic sanity check for reverse mapping process. This option is on by default. However, if output valid packet info file size optimization is desired, the expected scan out data may be removed from the output valid packet info file with this option off. This option will be effective only if commandline option “--outputValidPacketFile” is given for scan content transformation. Otherwse, this option will be ignored.

Syntax:

tool_set record_expected_data_in_outputValidPacketFile <on|off>;

Example:

tool_setup_options {

tool_set record_expected_data_in_outputValidPacketFile off;

}

#### stf_skip_bypass_hub_error_check

If an endpoint under a hub in bypass mode is accessed, SPF will produce an error by default. If it is desired to allow such a transaction especially for DFT validation, this tool option should be set on. This option is off by default.

Syntax:

tool_set stf_skip_bypass_hub_error_check <on|off>;

Example:

tool_setup_options {

tool_set stf_skip_bypass_hub_error_check on;

}

#### tap_ignore_name_case_sensitivity

The case sensitivity of TAP proper names such as TAP names, TAP register names, register field names, opcode names, mnemonics, interface pin names, register file module names and register file instance names can be ignored by turning on the following tool option. Likewise, any proper names for IJTAG resources such as register file instance names, register names and register field names are assumed to be case sensitive in SPF by default.

Syntax:

tool_set tap_ignore_name_case_sensitivity <on|off>;

Example:

tool_setup_options {

tool_set tap_ignore_name_case_sensitivity on;

}

This option is off by default.

#### stf_ignore_name_case_sensitivity

The case sensitivity of STF proper names such as STF stop names, STF register names, register field names, register alias/chain names, packet opcode names, testport name can be ignored by turning on the following tool option. This option is off by default.

Syntax:

tool_set stf_ignore_name_case_sensitivity <on|off>;

Example:

tool_setup_options {

tool_set stf_ignore_name_case_sensitivity on;

}

#### vector_compression

SPF can compress identical back-to-back STF vectors into a single vector with a repeat count. The threshold value is optional. If threshold value is not given in the option, its default value is 100. STF vectors with repeat count greater or equal to 100 will be compressed automatically.

Syntax:

tool_set vector_compression [optional_threshold_value];

Example:

tool_setup_options {

tool_set vector_compression 300;

}

#### stf_append_dummy_packet_to_test_end

SPF automatically enforces to return STF traffic to a non strobing state by the end of itpp file, when this option is set. Tool will add a number of dummy STF/TP packets (all 0’s for intput and X’s for output) at the end of test if the last vector written to ITPP is for STF/TP. User can specify the number of dummy packets they need to append. By default, this option is off and no additional packets is inserted.

Syntax:

tool_set stf_append_dummy_packet_to_test_end <packet_count>;

Example:

tool_setup_options {

tool_set stf_append_dummy_packet_to_test_end 1;

}

Itpp example:

STF: Additional dummy packets due to this option are highlighted

rem: in: stf_in : GID = 2 OPCODE = NOP Payload =

rem: out: REG = SCANDATA_REG STOPNAME = STF_ULT_PARHCAPSF stf_out : GID = 2 OPCODE = DATA Payload = XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

vector: stf_in(000000000000000000000000000000000010000001) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: stf_in : GID = 2 OPCODE = NOP Payload =

rem: out: REG = SCANDATA_REG STOPNAME = STF_ULT_PARHCAPSF stf_out : GID = 2 OPCODE = DATA Payload = XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX

vector: stf_in(000000000000000000000000000000000010000001) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

rem: in: stf_in : GID = 2 OPCODE = NOP Payload =

rem: out: REG = SCANDATA_REG STOPNAME = STF_ULT_PARHCAPSF stf_out : GID = 2 OPCODE = DATA Payload = XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXHL

vector: stf_in(000000000000000000000000000000000010000001) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXHLXXXXXXXXX);

rem: in: stf_in : GID = 0 OPCODE = NOP Payload =

vector: stf_in(000000000000000000000000000000000000000000) stf_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX);

TP: Additional dummy packets due to this option are highlighted

rem: in: tp_in : STF_CODE = 110 GROUP = 001 STF_DATA = 00000000000000011001 CONTROL_BITS = 11 INPUT_CONTROL = 1 OUTPUT_CONTROL = 0

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 10 INPUT_CONTROL = 1 OUTPUT_CONTROL = 1

vector: tp_in(001101000000000100100000000000111) tp_out(HLXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX) tp_ctrl1(01) tp_ctrl2(HH);

rem: in: tp_in : STF_CODE = 111 GROUP = 001 STF_DATA = 00000000000000001000 CONTROL_BITS = 11 INPUT_CONTROL = 1 OUTPUT_CONTROL = 0

rem: out: tp_out : STF_CODE = XXX GROUP = XXX STF_DATA = XXXXXXXXXXXXXXXXXXX CONTROL_BITS = 10 INPUT_CONTROL = 1 OUTPUT_CONTROL = 1

vector: tp_in(000101000000100000100000000000111) tp_out(HLXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX) tp_ctrl1(01) tp_ctrl2(HH);

rem: in: tp_in :

vector: tp_in(000000000000000000000000000000000) tp_out(XXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXXX) tp_ctrl1(00) tp_ctrl2(XX);

#### csr_ignore_name_case_sensitivity

The case sensitivity of  the name and attributes of CR Register File, register, and register field can be ignored by turning on the following tool option. This option is off by default.

Syntax:

tool_set csr_ignore_name_case_sensitivity <on|off>;

Example:

tool_setup_options {

tool_set csr_ignore_name_case_sensitivity on;

}

#### cr_error_on_inconsistent _crif

By default, SPF will ignore any discrepancy of a given CR CRIF file from the expected hierarchical structure of register file, register and field. However, if it is desired to flag such a discrepancy as an input error, this option should be turned on explicitly.

Syntax:

tool_set cr_error_on_inconsistent_crif <on|off>;

Example:

tool_setup_options {

tool_set cr_error_on_inconsistent_crif on;

}

Please refer to “CRIF File Consistency Check” section for the details of this option usage.

#### supress_csr_addr_conflict_error

By default, SPF will error out when it detects register address conflicts. Using this tool option user can deescalate errors to warnings, and tool will not abort when register address conflicts are found.

Syntax:

tool_set supress_csr_addr_conflict_error <on|off>;

Example:

tool_setup_options {

tool_set supress_csr_addr_conflict_error on;

}

## Default Directive Setting

The default values of directives can be given in a template section defined in the following text box.

directives {

<directive syntax defined in test seq>;

…

}

The same directive syntax defined in test sequence should be used as shown below.

@set attribute <value>;

The following directives are allowed to be set with their default values in the template file. Please refer to the document “Pattern Framework Spec - Test” for details on directives.

### Directive Option

For TAP:

| Directive | Impact |
| --- | --- |
| tap_auto_mode | Turns on/off automatic tap/enabling or disabling of taps in the hierarchical tap structure. Insignificant for tap link |
| tap_overshift_mode | Enables/disables overshift mode. Not supported for tap link |
| tap_incremental_mode | Enables or disables incremental mode of setting bits of register |
| tap_opcode | Sets default op-code for a register. Not supported for glues |
| tap_auto_unfocus | Used to turn off default disabling of taps in auto mode at the end of test sequence. By default, if auto mode is on, all taps will be disabled at the end of test sequence. However, if manual mode is turned on, automatic tap disabling will be turned off. Insignificant for tap link. |
| tap_elaborate_label | Enables/disables elaborate labeling for shift IRs/DRs with tap name, opcode, registername and offset information. |
| tap_ir_label | Enables/disables ir labels |
| tap_skip_ir | Skips IR shift for a register in case the register was previously accessed where the ir was shifted |
| tap_auto_retime | Enables/disables automatic addition of re-time taps to the sequence |
| tap_pscan_delay | Sets pscan delay value in the case of TDO on NOA. Per pin delay specification not allowed through template. |
| tap_allow_capture_high_mode | Enables capturing output with High value. |
| tap_skip_dr_back_padding | Skips DR low padding for a register in case there is no data strobe on the register. |
| tap_post_dr_delay_override | Overrides post dr delay in tap link network |
| tap_disable_control_bit_incremental_mode | Disables incremental mode for SELECT register, dynamic/static registers constituting control bits for configuring another register and a CONFIG register in a TAP link EP. |
| tap_auto_register_layout_config | Enables automatic programming of IJTAG network |
| tap_segmented_dr_shift_adjusted_for_tap_link | Automatically adjust the first shift segment to account for the front pad bits in segmented DR shift in TAP link network. |
| tap_ijtag_optimize_sib_config | Optimize SIB network programming. |

For STF:

| stf_wide_register_specified_access | By default is off and syntax shown as below: @set stf_wide_register_specified_access <off ¦ WITH_ACTION ¦ WITHOUT_ACTION> ; - Off - Generates multiple packets containing complete register value - WITH_ACTION - Generates packets with specified bits in wide register with a packet for register action address. - WITHOUT_ACTION - Only generate packets with specified bits accessed in wide register |
| --- | --- |
| stf_flush_output_packets | By default is on and syntax shown as below: @set stf_flush_output_packets <on ¦ off>; To control auto flush out capability. |
| stf_incremental_mode | Enables or disables incremental mode of setting bits of register |
| stf_serial_reg_broadcast | By default is off and syntax shown as below: @set stf_serial_reg_broadcast <on ¦ off>; Enables or disables broadcasting a serial wide register transaction to multiple STF Eps even if the widths of the register varies among the endpoints in the same group |
| stf2Tap_input_expect_mode | By default is off and syntax shown as below: @set stf2Tap_input_expect_mode on ¦ off; - Off: regular input/output mode is set during STF2TAP transformation - On: input/expect mode is set during STF2TAP transformation |
| stf2Tap_assert_shift_count | By default is off and syntax shown as below: @set stf2Tap_assert_shift_count on ¦ off; - On: assert shift_count field in SHIFT mode, and also shift_count data will be included in expect data if expect mode is set - Off: do not assert shift_count field in SHIFT mode |
| tap_2_stf | @set tap_2_stf <STF stop> ¦ <entry STF stop>, <exit STF stop> Swtich to a TAP2STF mode and specifies the entry and exit componenets to a partial STF network in TAP2STF mode |
| tap_2_stf_off | Switch back to normal STF mode from TAP2STF mode. |

Set_transform and Set_template directives:

A directive to set “transform” and “template” defined in template file is available in the template file. Please refer to “Directive to Set Template and Transform” section in “Pattern Framework Spec-Test” document for the details about directive.

| @set_template <test access type> <Template Name> | Sets template corresponding to the Template Name specified |
| --- | --- |
| @set_transform <destination access type> <source access type> <Transform Name> | Sets transform corresponding to the Transform Name specified |
| @set_template <test access type> <Template Name>, TEMPLATE_VARIABLES = {<comma separated user defined variables>} | In case parameterized template is selected, all the template variables defined in the parameterized template should be initialized in the directive |
| @set_transform <destination access type> <source access type> <Transform Name>, TEMPLATE_VARIABLES = {<comma separated user defined variables>} | In case parameterized transform is selected, all the variables defined in the parameterized transform should be initialized in the directive |

# Template File Override

A common template file is typically prepared at the project level and shared across all the content development teams for their content generation purpose. However, certain content owners may need to have content specific template information which is different from or in addition to the common project level template. To satisfy this requirement, SPF supports the override of the default template file with a content specific template file. In addition to the default template file, a template override file to override the information in the default template file may be given to SPF as an input using a commandline option “--templateOverrideFile <file name>”.

If a template file is given with the override option (--templateOverrideFile) while the default template option (--templateFile) is not given, the given template file will be processed in the same way as it is given with the default template option.

## Override Behavior

The override behavior for each component of template file is described in the following subsections.

### Tool Option and Default Directive Setting Override

Tool options and default directives settings given in the default template file are merged with those given in the override file.

If the same options are given in the two files and they have conflicting value assignments, warning messages will be reported and the options will be set with the values given in the override file.

Consider the following example.

Template File:

tool_setup_options {

tool_set convert_internal_remark_to_comment off;

tool_set handle_duplicate_label uniquify;

tool_set tap_ignore_name_case_sensitivity on;

tool_set supress_csr_addr_conflict_error off;

}

directives {

@set tap_incremental_mode off;

@set_transform_seq TAP CR::GPSB TAP2IOSF,

TEMPLATE_VARIABLES = {

$TAP = stap_tap2gpsb

};

@set_template TAP mmd_misc_shifttmpl, TEMPLATE_VARIABLES = {@DR_SHIFT = {10, 8, 27, 8, 'REMAINING_DR_SHIFT}, $CYCLE = 1};

}

Template Override File:

tool_setup_options {

tool_set convert_internal_remark_to_comment on;

tool_set allow_multiple_register_per_opcode on;

tool_set handle_duplicate_label uniquify;

tool_set supress_csr_addr_conflict_error on;

tool_set csr_ignore_name_case_sensitivity on;

}

directives {

@set tap_incremental_mode on;

@set csr_incremental_mode on;

@set_transform_seq TAP CR::GPSB TAP2IOSF_1,

TEMPLATE_VARIABLES = {

$TAP = stap_tap2gpsb

};

@set_transform_seq TAP CR::PMSB TAP2IOSF_1,

TEMPLATE_VARIABLES = {

$TAP = stap_tap2pmsb

};

@set_template TAP mmd_misc_shifttmpl_1, TEMPLATE_VARIABLES = {@DR_SHIFT = {10, 8, 27, 8, 'REMAINING_DR_SHIFT}, $CYCLE = 1};

}

The final list of options considered after merging two template files:

tool_setup_options {

tool_set convert_internal_remark_to_comment on; //value for this options considered from override file

tool_set allow_multiple_register_per_opcode on; //option specified only in override file

tool_set handle_duplicate_label uniquify; //option specified in both the files with the same value

tool_set tap_ignore_name_case_sensitivity on; //option specified only in default template file

tool_set supress_csr_addr_conflict_error on;	// value for this options considered from override file

tool_set csr_ignore_name_case_sensitivity on; // option specified only in override file

}

directives {

@set tap_incremental_mode on; // value for this options considered from override file

@set csr_incremental_mode on; //option specified only in override file

@set_transform_seq TAP CR::GPSB TAP2IOSF_1,

TEMPLATE_VARIABLES = {

$TAP = stap_tap2gpsb

};				//option specified in override file with different transform seting

@set_transform_seq TAP CR::PMSB TAP2IOSF_1,

TEMPLATE_VARIABLES = {

$TAP = stap_tap2pmsb

};					// option specified only in override file

@set_template TAP mmd_misc_shifttmpl_1, TEMPLATE_VARIABLES = {@DR_SHIFT = {10, 8, 27, 8, 'REMAINING_DR_SHIFT}, $CYCLE = 1};   //default directive considered from override file.

}

### TEMPLATE/TRANSFORM Defintion Override

TEMPLATE and TRANSFORM definitions given in the default template file are merged with those given in the override file. If a TEMPLATE or TRANSFORM defined in the default template is redefined in the override file, a warning message will be reported and the definition given in the override file will be considered.

Please note that each TEMPLATE or TRANSFORM should be defined with a unique name. If two different TEMPLATEs/TRANSFORMs are defined with the same name, an error will be reported.

Consider the following example.

Template File:

TRANSFORM STF MCI MCI2STF {

// Initial steps at the start of MCI stream

PRE_AMBLE = {

set_stf_packet input SELECT 1 'h0c 0;

focus_stf  pardmi_scanunit_ep;

set pardmi_scanunit_ep=>MODE->gid0_output_enable='b1;

flush;

}

// Main steps for MCI to STF transformation

STEPS = {

LOCAL_VAR = {

$pkt1   = 0;

$pkt2   = 1;

}

focus_stf pardmi_scanunit_ep;

set CHAIN_DATA = PACKET::MCI_IN{0}[19:0];

compare CHAIN_DATA = PACKET::MCI_OUT{0}[19:0];

flush;

focus_stf pardmi_scanunit_ep;

set CHAIN_DATA = PACKET::MCI_IN{1};

compare CHAIN_DATA = PACKET::MCI_OUT{1};

flush;

comment "end_of_steps";

}

POST_AMBLE = {

}

}

Template Override File:

TRANSFORM STF MCI MCI2STF {

DELAY = {

MCI : 2;

STF : 3;

}

// Initial steps at the start of MCI stream

PRE_AMBLE = {

set_stf_packet input SELECT 1 'h0c 0;

focus_stf  pardmi_scanunit_ep;

set pardmi_scanunit_ep=>MODE->gid0_output_enable='b1;

flush;

}

// Main steps for MCI to STF transformation

STEPS = {

LOCAL_VAR = {

$pkt1   = 0;

$pkt2   = 1;

}

focus_stf pardmi_scanunit_ep;

set CHAIN_DATA = PACKET::MCI_IN{0}[19:0];

compare CHAIN_DATA = PACKET::MCI_OUT{0}[19:0];

flush;

focus_stf pardmi_scanunit_ep;

set CHAIN_DATA = PACKET::MCI_IN{1};

compare CHAIN_DATA = PACKET::MCI_OUT{1};

flush;

comment "end_of_steps";

}

POST_AMBLE = {

}

}

In the above example, the TRANSFORM ‘MCI2STF’ is defined in the default template and the override file. The TRANSFORM definition in the override file with delay value specification will be considered for further processing.

### Project Spec Override

tap_network_list given in the default template file is merged with the one given in the override file.

Consider the following example.

Template File:

PROJECT_SPEC {

tap_network_list {

prim;

sec;

}

}

Template Override File:

PROJECT_SPEC {

tap_network_list {

prim;

ter;

}

}

The final tap_network_list considered after merging two template files.

PROJECT_SPEC {

tap_network_list {

prim; //network name specified in both the files

sec; //network name specified only in template file

ter;  //network name specified only in override file

}

}

Each section under TAP_SPEC_AUGMENT given in the default template file is merged with the one given in the override file. If select override information is given in both files for the same TAP, a warning will be reported and the information given in the override file will be considered for the given TAP. The same process applies to TAP_AUGMENT section. If TAP augment information is provided in both default template and override file for the same TAP, a warning will be reported and the information given in the override file will be considered for the given TAP.

Consider the following example.

Template file:

project_spec {

TAP_SPEC_AUGMENT {

SELECT_OVERRIDE {

CNV_WIFI_APMG_TAP {

NORMAL {

CNVI_TOP_TAP=>reg_jtag_reset[2] = 'b1;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = 'b10;

CNVI_TOP_TAP=>reg_jtag_tap_bypass[3] = 'b0;

}

ISOLATE {

CNVI_TOP_TAP=>reg_jtag_tap_bypass = 'DEFAULT;

DISABLED;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = 'DEFAULT;

CNVI_TOP_TAP=>reg_jtag_reset[2] = 'b0;

}

}

CNV_MFU_TAP {

NORMAL {

CNVI_TOP_TAP=>reg_jtag_reset[1] = 'b1;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = 'b10;

CNVI_TOP_TAP=>reg_jtag_tap_bypass[2] = 'b0;

}

ISOLATE {

CNVI_TOP_TAP=>reg_jtag_tap_bypass = 'DEFAULT;

DISABLED;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = 'DEFAULT;

CNVI_TOP_TAP=>reg_jtag_reset[1] = 'b0;

}

}

}

}

Template Override File:

project_spec {

TAP_SPEC_AUGMENT {

SELECT_OVERRIDE {

CNV_JTAG_PIPE_TAP {

NORMAL {

CDU_CNVI_TAP=>TAPC_SELECT[0] = 'b1;

}

ISOLATE {

CDU_CNVI_TAP=>TAPC_SELECT[0] = 'b0;

}

}

CNV_WIFI_APMG_TAP {

NORMAL {

CNVI_TOP_TAP=>reg_jtag_reset[2] = 'b0;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = 'b10;

CNVI_TOP_TAP=>reg_jtag_tap_bypass[4] = 'b0;

}

ISOLATE {

CNVI_TOP_TAP=>reg_jtag_tap_bypass = 'DEFAULT;

DISABLED;

CNVI_TOP_TAP=>reg_jtag_tck_cg_en[1:0] = 'DEFAULT;

CNVI_TOP_TAP=>reg_jtag_reset[2] = 'b1;

}

}

}

TAP_AUGMENT {

CNVI_TOP_TAP

{

INDIRECT_REGISTERS {

reg_jtag_tck_cg_en {

@SIZE = 2;

@ADDRESS = 'h80000008;

@default = 'b00;

}

reg_jtag_reset {

@SIZE = 6;

@ADDRESS = 'h80000004;

@default = 'b000000;

}

reg_jtag_tap_bypass {

@SIZE = 14;

@ADDRESS = 'h80000000;

@default = 'h3ffe;

}

}

INDIRECTION_COMMANDS {

WRITE {

DEFAULT {

set ADDRESS = REGISTER->ADDRESS; flush;

set DATA = REGISTER->VALUE; flush;

set TRANSCMD = 'h01; flush;

}

}

READ {

DEFAULT {

set CNVI_TOP_TAP=>ADDRESS = REGISTER->ADDRESS; flush;

set CNVI_TOP_TAP=>TRANSCMD = 'h05; flush;

compare CNVI_TOP_TAP=>DATA = REGISTER->VALUE; flush;

}

}

}

}

}

}

In the above example, the SELECT_OVERRIDE definition for CNV_WIFI_APMG_TAP is given in both the files. In this case, the definition in the override file will be considered. However, select override definitions for CNV_MFU_TAP and CNV_JTAG_PIPE_TAP are given only in one of the two files. In this case, the definitions for the given two TAPs are considered as part of the finally merged template. TAP_AUGMENT section for CNVI_TOP_TAP is given in the override file only. So it will be considered as part of the finally merged template.

cr_spec section given in the default template file is merged with the one given in the override template file. If the same CR fabric type is defined in both files, the definition given in the override template file will be used while the one given in the default template file is ignored.

Consider the following example.

Template file:

PROJECT_SPEC {

cr_spec {

IOSF_SB {

OPCODE_ENUM_MAP {

MEM-SB = 'h00 (read) : 'h01 (write);

IO-SB  = 'h02 (read) : 'h03 (write);

CFG-SB = 'h04 (read) : 'h05 (write);

CR-SB  = 'h06 (read) : 'h07 (write);

}

REGISTER_SPACES {

GPSB {

OPTIONAL {

'SB_FID {

spec = REG_FILE::fid;

default = 0;

}

}

QUALIFY {

A : 'REG_FILE_TYPE = MSG;

B : 'FABRIC = GPSB;

C : 'PORTID = {48,49};

CONDITION : A && B && !C;

}

}

PMSB {

QUALIFY {

A : 'REG_FILE_TYPE = MSG;

B : 'FABRIC = PMSB;

C : 'PORTID = {48,49};

CONDITION : A && B && !C;

}

}

}

}

}

}

Template Override file:

PROJECT_SPEC {

cr_spec {

IOSF_SB {

OPCODE_ENUM_MAP {

MEM-SB = 'h00 (read) : 'h01 (write);

IO-SB  = 'h02 (read) : 'h03 (write);

CFG-SB = 'h04 (read) : 'h05 (write);

CR-SB  = 'h06 (read) : 'h07 (write);

}

REGISTER_SPACES {

GPSB {

OPTIONAL {

'SB_FID {

spec = REG_FILE::fid;

default = 1;

}

}

QUALIFY {

A : 'REG_FILE_TYPE = MSG;

B : 'FABRIC = GPSB;

C : 'PORTID = {48,49};

CONDITION : A && B && !C;

}

}

PMSB {

QUALIFY {

A : 'REG_FILE_TYPE = MSG;

B : 'FABRIC = PMSB;

C : 'PORTID = {48,49};

CONDITION : A && B && !C;

}

}

}

}

}

}

In the above example, IOSF_SB fabric type definition in cr_spec is given in both the files. However the definition in the override file will be considered as part of the finally merged template.